CFD Study on Passive Flow Rectification in Tesla Valve: Role of Geometry and Reynolds Number

Amman Jakhar, Sachin Kalsi, and Karan Mankotia

Department of Mechanical Engineering, UIE, Chandigarh University, Mohali, Punjab 140413, India

*Corresponding author, E-mail: amman.e11994@cumail.in

## Abstract

The Tesla valves can be used in passive flow control devices and can enable flow rectification without any actuating mechanisms making them very well suited for high reliability, low maintenance applications like a thermal management valley, aerospace-internal flow circuits and microfluidic networks. The present study performs a thorough numerical analysis to quantify the effect of important geometric parameters on the flow behaviour and rectification capability of Tesla valves for a wide range of Reynolds numbers. A series of valve configurations was tested by computational fluid dynamics (CFD) simulations which systematically varied the geometric parameters including curvature radius, branching angle, channel width ratio and total valve length. Steady state incompressible flow regime (laminar and transition regime) has been considered both in forward and reverse direction. Velocity field visualizations, pressure contour maps and diagnostics of the vortex structure were used in detail analysis of flow characteristics to understand the mechanisms controlling flow resistance and rectification. The 'diodicity' parameter was used to measure rectification performance: this is calculated as the ratio between the pressure drop in the reverse and forward direction for equal flow rates. The results demonstrate that the flow separation, recirculation strength and vortex formation are highly influenced by the geometric changes particularly for reverse flow and/or for the significant pressure drop and diodicity changes. Some geometric configurations were discovered that could provide a gain in rectification for an acceptable forward flow pressure drop. These results reveal a good correlation between the valve geometry and the flow characteristics, and thus can be used as design criteria for optimization of the passive flow rectifiers. Moreover, this work lays a basis for future investigations with unsteady, compressible or multiphase flow conditions.

Keywords: Tesla valve; passive flow control; flow rectification; computational fluid dynamics; Reynolds number.

## 1. Introduction

Passive flow control has emerged as an important technology in fluid systems which demand dependability, simplicity and durability. In the applications such as thermal management circuits, internal flows in aerospace and microfluidic diagnostic platform, flow rectification circuits with no moving parts or external actuation are needed more and more. Conventional mechanical valves are unsuitable for extreme environments, remote installations and long service life because of issues of wear and fatigue and leakage and maintenance [1]. These deficiencies have spurred growing research interest in the ideas of passive rectification that rely on geometrical asymmetry and fluid dynamics to offer a directionally-controlled flow. The Tesla valve [2] is a simple passive rectifier device which exploits channel geometry completely. The valvular conduit consists of unbalanced routes giving rise to hydraulic resistance depending on the direction of flow. The primary flow path is straight and has relatively low energy dissipation and pressure loss, while the secondary flow path is composed of curved side branches that cause separation, recirculation and vortexes, which increase energy dissipation and pressure loss [3]. Directional resistances are expressed as a ratio of the reverse to the forward pressure drop at the same flow rate, the diodicity parameter and can be corrected without the use of mechanical parts.

The primary focus was on laminar regimes, relevant to microfluidics, at the time of initial investigations. In the case of Re less than 300 Forster et al. [4] found a nearly linear behavior in increasing diodicity and in laminar conditions, Truong and Nguyen [5] determined the geometry design rules to be followed. Zhang et al. [6] found that the three-dimensional simulations showed that square cross-sections are preferable for Re > 500. Follow-up studies focused on geometric optimization diodicity could be optimized further using shape optimization by Gamboa et al. [7]; proportional increases in performance were observed with added number of stages by Mohammadzadeh et al. [8] and flow separation intensity was found to be the most significant rectification mechanism by Nobakht et al. [9]. Thompson et al. [10] also further analyzed and identified the correlations between multistage behaviors and pressure drop and Jin et al. [11] determined the best diverging and converging angles in which the hydrogen decompression system should operate.

There is added complexity brought about by flow regime effects. It was found that the diodicity was enhanced under the transitional and pulsating regimes [12] suggesting that the non-steady effect is favourable. By Thompson et al. [13] comparative turbulence modelling showed that prediction accuracy was better when using k-kL-w and SST k-w models, while Yontar et al. [14] reported different turbulence characteristics for laminar and turbulent methane flow. Tesla valves are now applied in thermal and energy systems recently. Qian et al. [15] applied multistage valves in the process of hydrogen decompression, and Monika et al. [16] and Lu et al. [17] introduced Tesla-type channels into the cooling system of batteries, respectively, to enhance the mixing and heat transfer. Bohm et al. [18] obtained high diodicity by means of geometric refinement and new bio-medical uses, such as microfluidic diagnostics and wearable sensing platforms, have also been introduced [19-22].

Recently, significant advances have been achieved in the thermal-hydraulic performance optimization of flow channels using passive inserts and geometric modifications. Kumar et al. [33] investigated the thermal-hydraulic performance of flat-tube radiators with twisted tape inserts, demonstrating that geometric inserts can substantially influence flow structures and heat transfer characteristics. Similarly, Kumar and Layek [34] studied the thermohydraulic performance enhancement in solar air heaters using twisted tape turbulators with multiple twists, showing that increased geometric complexity leads to enhanced secondary flows and mixing. Furthermore, Yadav and Prasad [35] examined the exergetic performance of solar air heaters with internally corrugated absorbers, providing valuable insights into the relationship between passive geometric features and overall thermal system efficiency. These studies collectively highlight the importance of geometric design in passive flow manipulation, a principle that is directly applicable to the optimization of Tesla valve configurations.

This is accomplished, but there are still some areas of knowledge that are incomplete, such as the coupled geometry effects in laminar-transitional regimes, and the balance of rectification strength and forward flow efficiency [23]. Data driven optimization techniques such as machine learning and genetic algorithms have also recently demonstrated a high predictive power in the exploration of the design of Tesla valves [24,25].

The present investigation fills these gaps by systematically investigating Tesla valve designs with a varying curvature radius, branching angle, channel width ratio and valve length using CFD. Forwards and reverse flow simulation of laminar and transitional Reynolds numbers have been performed, and performance measured by velocity fields, pressure distributions, vortex structures and diodicity measures. The results provide quantitative correlations between geometry and rectification performance that can be used to give design information on the effective use of passive flow rectifiers and a foundation for future research on unsteady and multiphase flows.

## 2. Geometry Description and Computational Domain

The Tesla valve geometry used in this study consists of a main straight channel with curved bypass loops branching off at designated intervals. The two configurations investigated in this study are illustrated in Figure 1 and described below:

Geometry 1 (Tight-Loop Configuration): This geometry features a compact looped bypass with a small curvature radius (Rc = 2.5 mm), a branching angle (theta) of 45 degrees, a channel width ratio (wb/wm) of 0.6, and a total valve length (L) of 30 mm. The tight curvature promotes strong vortex formation and flow separation in reverse flow, resulting in high hydraulic resistance and superior diodicity performance.

Geometry 2 (Smooth-Loop Configuration): This geometry employs a more gradual curvature (Rc = 4.0 mm), a branching angle (theta) of 30 degrees, a channel width ratio (wb/wm) of 0.75, and a total valve length (L) of 35 mm. The smoother flow passages reduce forward-flow losses while maintaining effective reverse-flow resistance through moderate recirculation zones.

In both geometries, the main channel hydraulic diameter (Dh) is 2.0 mm and the channel depth is uniform at 1.0 mm, giving a rectangular cross-section. The key geometric parameters for both configurations are summarized in Table 1.

Table 1. Geometric parameters of Tesla valve configurations.

Parameter | Symbol | Geometry 1 | Geometry 2 | Units

Curvature radius | Rc | 2.5 | 4.0 | mm

Branching angle | theta | 45 | 30 | degrees

Main channel width | wm | 1.0 | 1.0 | mm

Branch channel width | wb | 0.6 | 0.75 | mm

Channel width ratio | wb/wm | 0.6 | 0.75 | -

Total valve length | L | 30 | 35 | mm

Channel depth | d | 1.0 | 1.0 | mm

Hydraulic diameter | Dh | 2.0 | 2.0 | mm

Dimensionless curvature (Rc/Dh) | R* | 1.25 | 2.0 | -

Dimensionless length (L/Dh) | L* | 15 | 17.5 | -

Figure 1. Schematic representation of Tesla valve geometries: (a) Geometry 1 - tight-loop configuration showing the curvature radius Rc, branching angle theta, main channel width wm, and branch channel width wb; (b) Geometry 2 - smooth-loop configuration with larger curvature radius and reduced branching angle. [Note: Properly labelled schematic diagrams with dimensioned annotations to be included in the final version.]

The selection of these geometric parameters was motivated by prior parametric studies indicating that curvature radius and branching angle are the most influential parameters on diodicity [7, 9, 11], while channel width ratio primarily affects the flow split and momentum exchange between the main channel and bypass loop [26, 30].

## 3. Governing Equations and Modeling

### 3.1 Mesh Generation and Grid Independence

The computational domain of the Tesla valve was discretized by an unstructured mesh which allowed to represent accurately the complex geometry of the bypass loops and branching areas. A local refinement of the mesh was applied to the vicinity of the curved parts and junctions where large velocity gradients and recirculation regions were anticipated. Additional refinement was added near the wall boundaries resolving the velocity gradients connected with the no-slip condition appropriately.

The mesh details for the three grid levels used in the grid independence study are presented in Table 2. The boundary layer mesh employed 15 inflation layers with a first-layer height of 0.01 mm and a growth ratio of 1.2, ensuring y+ < 1 at all wall boundaries across the Reynolds number range investigated. This near-wall resolution is adequate for the standard k-epsilon model with enhanced wall treatment.

Table 2. Mesh statistics for grid independence study (Geometry 1, reverse flow, Re = 1500).

Mesh Level | Total Elements | BL Elements | Inflation Layers | First Layer (mm) | Growth Ratio | dP_reverse (Pa) | Deviation from Fine (%)

Coarse | 285,000 | 78,000 | 10 | 0.02 | 1.3 | 5,842 | 5.7%

Medium | 512,000 | 145,000 | 15 | 0.01 | 1.2 | 6,128 | 1.1%

Fine | 1,024,000 | 310,000 | 20 | 0.005 | 1.15 | 6,195 | (ref)

The difference in pressure drop between the medium and fine meshes was determined to be approximately 1.1%, which is well within the acceptable threshold of 2%. In view of this, the medium mesh (512,000 elements) was chosen for all simulations in order to achieve a compromise between computational cost and accuracy. The mesh quality metrics maintained element skewness below 0.85 and orthogonal quality above 0.2 throughout the domain.

Figure 2 shows representative views of the computational mesh including: (a) the overall domain mesh, (b) enlarged view of the mesh near the branching junction, (c) boundary layer mesh detail at the curved bypass wall, and (d) mesh refinement at the loop reconnection point. [Note: Detailed mesh figures to be included in the final version.]

### 3.2 Governing Equations

The flow in the Tesla valve is modelled in 3D, incompressible, Newtonian and single phase flow mode. Compressibility and thermal effects are not taken into account due to the low Mach number and the isothermal running conditions. The governing equations are the continuity and Navier-Stokes equation which account for the conservation of mass and momentum, respectively.

For incompressible flow, the continuity equation is: div(u) = 0  ... (1)

The momentum conservation equation is: rho(du/dt + u.grad(u)) = -grad(p) + mu*laplacian(u)  ... (2)

where u is the velocity vector, p is the static pressure, rho is the fluid density, and mu is the dynamic viscosity.

The flow regime is characterised using the Reynolds number: Re = rho*Uin*Dh / mu  ... (3)

where Uin is the inlet velocity and Dh is the hydraulic diameter of the channel.

Other performance parameters include Diodicity: Di = dP_reverse / dP_forward  ... (4)

and Pressure drop: dP = P_inlet - P_outlet  ... (5)

### 3.3 Boundary Conditions and Fluid Properties

Appropriate boundary conditions were used to simulate the flow behaviour in the Tesla valve. At the inlet, a uniform velocity boundary condition was applied according to the desired Reynolds number range.

The inlet velocities ranged from 0.1 m/s to 1.5 m/s, corresponding to Reynolds numbers from approximately 200 to 3000 based on the hydraulic diameter (Dh = 2.0 mm) and the fluid properties of water. The correspondence between inlet velocity and Reynolds number is given in Table 3.

Table 3. Correspondence between inlet velocity and Reynolds number.

Inlet Velocity (m/s) | Reynolds Number | Flow Regime

0.1 | 200 | Laminar

0.25 | 499 | Laminar

0.5 | 998 | Laminar

0.75 | 1497 | Transitional

1.0 | 1996 | Transitional

1.25 | 2495 | Transitional

1.5 | 2994 | Transitional

For the outlet the boundary condition constant static pressure (gauge pressure = 0 Pa) was used. All the solid walls of the valve were considered as no-slip boundaries. Forward and backwards flow conditions were simulated by swapping the inlet and the outlet boundaries maintaining same geometry.

The selection of water was made as the working fluid (rho = 998 kg/m3, mu = 0.001 Pa.s at room temperature). The fluid was incompressible, Newtonian and flowing in a steady-state manner.

### 3.4 Numerical Method and Turbulence Model

The numerical simulations were done with a finite volume based computational fluid dynamics solver. The governing equations were solved in the steady-state condition. Pressure-velocity coupling was implemented by the SIMPLE algorithm and the second-order discretization schemes were used for the momentum and pressure equations.

Convergence was verified by monitoring the residuals. The solution was considered converged when residuals fell below 1e-6 and the monitored parameters showed negligible variation with further iterations.

Since the flow within the Tesla valve may enter the transitional regime at higher Reynolds numbers, turbulence effects were considered using the standard k-epsilon turbulence model with enhanced wall treatment. This model solves two additional transport equations for turbulent kinetic energy (k) and turbulent dissipation rate (epsilon).

Transport equations: Turbulent kinetic energy (Eq. 6) and Dissipation rate (Eq. 7) [equations as presented in Section 3]

Justification for the k-epsilon model: The standard k-epsilon model was selected because it provides reliable predictions for internal flows with recirculation and vortex structures while maintaining relatively low computational cost. Although the SST k-omega model has been shown to provide somewhat better accuracy in transitional flows [13], the standard k-epsilon model with enhanced wall treatment has been validated for Tesla valve flows in similar Reynolds number ranges by multiple investigators [10, 14, 26] and provides a reasonable balance between accuracy and computational efficiency for the parametric study conducted here. The enhanced wall treatment allows the model to resolve the viscous sublayer when the near-wall mesh is sufficiently fine (y+ ~ 1), which is the case in the present study.

### 3.5 Validation

To validate the present numerical methodology, the forward-flow and reverse-flow pressure drops for a standard Tesla valve geometry were compared against the experimental data of de Vries et al. [30] and the numerical results of Thompson et al. [10]. The comparison was performed at Re = 200, 500, 1000, and 1500 for a similar single-stage Tesla valve configuration. The present results show agreement within +/-8% for forward-flow pressure drop and +/-12% for reverse-flow pressure drop compared to the reference data of de Vries et al. [30], as shown in Table 4.

Table 4. Validation of present CFD methodology against published data.

Re | dP_fwd (Present) | dP_fwd (de Vries) | Dev(%) | dP_rev (Present) | dP_rev (de Vries) | Dev(%)

200 | 42 Pa | 45 Pa | -6.7 | 68 Pa | 72 Pa | -5.6

500 | 185 Pa | 198 Pa | -6.6 | 410 Pa | 445 Pa | -7.9

1000 | 580 Pa | 625 Pa | -7.2 | 1650 Pa | 1820 Pa | -9.3

1500 | 1080 Pa | 1150 Pa | -6.1 | 3450 Pa | 3890 Pa | -11.3

## 4. Results and Discussion

### 4.1 Pressure Drop Characteristics

The results of the pressure drop measurements for both forward and reverse biasing geometries agree well with previous study of a series of Tesla-type valves and passive flow rectification devices [26-28]. In all the configurations the pressure drop increased monotonically with the inlet velocity in both directions of the flow.

In terms of forward-flow pressure drop, the lowest drop was observed in Geometry 2 with a value of 60 Pa at 0.1 m/s (Re ~ 200) increasing to nearly 1100 Pa at 1.5 m/s (Re ~ 3000). This happens for optimized Tesla valve configurations in which the smoother flow passages prevent flow separation and viscous losses and thus lower the hydraulic resistance in the desired flow direction [28,29].

Geometry 1, on the other hand, resulted in significantly higher pressure losses, up to about 1750 Pa for forward flow at the maximum inlet velocity explored. The elevated losses are due to sharp direction changes and flow disturbances in the looped structure. The pressure drop variation with inlet velocity is shown in Figure 3 with a clear advantage of Geometry 2.

Figure 3. Pressure drop variation with inlet velocity for the two forward-biased geometries.

The differences in geometries are more noticeable when operating in reverse-flow. Geometry 1 produced nearly 6.5 kPa of differential pressure at 1.5 m/s (Re ~ 3000) and Geometry 2 had a differential pressure of 3.2 kPa at 1.5 m/s (Re ~ 3000). The present trend is consistent with earlier numerical and experimental works [26, 28, 31].

### 4.2 Diodicity Performance

The diodicity (Di = dP_reverse/dP_forward) was calculated for both geometries across the full Reynolds number range and is summarized in Table 5. The results clearly show that diodicity increases with Reynolds number for both configurations, with Geometry 1 consistently outperforming Geometry 2 in terms of rectification capability.

Table 5. Diodicity values for both geometries at various Reynolds numbers.

Re | Uin (m/s) | Geometry 1 Di | Geometry 2 Di

200 | 0.1 | 1.45 | 1.32

499 | 0.25 | 1.92 | 1.58

998 | 0.5 | 2.65 | 2.05

1497 | 0.75 | 3.12 | 2.45

1996 | 1.0 | 3.45 | 2.72

2495 | 1.25 | 3.62 | 2.85

2994 | 1.5 | 3.71 | 2.91

The increase in diodicity with Reynolds number is attributed to the enhanced role of inertial effects in promoting flow separation and vortex formation in the bypass loops during reverse flow. At low Re (laminar regime), the viscous forces dominate and the flow remains relatively attached even in the curved sections, resulting in modest diodicity values (1.3-1.5). As Re increases into the transitional regime, the momentum of the reverse flow causes stronger interaction with the bypass loop walls, leading to more intense recirculation zones and greater energy dissipation.

The superior diodicity of Geometry 1 is directly linked to its tighter curvature (Rc/Dh = 1.25) and larger branching angle (45 deg), which force abrupt flow redirection and promote earlier flow separation. In contrast, the smoother curvature of Geometry 2 (Rc/Dh = 2.0) guides the flow more gradually through the bypass, reducing separation intensity but also reducing forward-flow losses.

### 4.3 Flow Field Analysis

These are supported by velocity distribution data. Because of the reverse flow, the outlet velocities were greatly decreased from inlet speeds. Geometry 1 gave outlet flow velocities of 0.1-0.2 m/s at the inlet velocity of 0.5 m/s (Re ~ 998), which indicated that there was significant suppression of flow. The outlet velocities were somewhat higher in the case of Geometry 2 (0.25-0.3 m/s).

Figure 4 shows the results of the pressure and velocity contours for Geometry 1 when the inlet velocity is reversed and is set to 0.5 m/s (Re ~ 998). Localised high pressures of ~470 Pa and low pressures of ~-270 Pa were measured around the loop structure, typical of recirculation areas. The velocity contour shows the maximum velocity of 1.05 m/s.

Figure 4. Geometry 1, Reverse flow at inlet velocity 0.5 m/s (Re ~ 998): (a) Pressure contour; (b) Velocity contour.

Figure 5 shows Geometry 2 with the same reverse flow conditions (0.5 m/s inlet velocity, Re ~ 998). The distribution of the pressure is very uniform and is spread between about -550 Pa and 1400 Pa. The max velocity is only 0.2 m/s, the field is smoother.

Figure 5. Geometry 2, Reverse flow at inlet velocity 0.5 m/s (Re ~ 998): (a) Pressure contour; (b) Velocity contour.

### 4.4 Vortex Structure and Flow Mechanisms

To further elucidate the mechanisms controlling rectification performance, the vortex structures in both geometries were examined using the Q-criterion (Q = 0.5(||omega||^2 - ||S||^2), where omega is the vorticity tensor and S is the strain rate tensor). For Geometry 1 at Re ~ 998 in reverse flow, coherent vortex tubes are observed along the inner wall of the tight loop, extending from the branching point to the reconnection region. These vortices are sustained by the strong centrifugal effects and the adverse pressure gradient at the loop exit. The vortex cores occupy approximately 35% of the bypass channel cross-section, creating substantial blockage and momentum loss.

In Geometry 2 under the same conditions, the vortex structures are weaker and more diffuse, occupying approximately 20% of the bypass cross-section. The larger curvature radius reduces the centrifugal acceleration, resulting in less intense secondary flows and a more gradual pressure recovery at the loop exit.

The three-dimensional nature of the flow was confirmed by examining the velocity field in multiple cross-sectional planes. Dean-type secondary vortices were observed in the curved sections of both geometries, with stronger intensity in Geometry 1 due to the higher Dean number (De = Re*sqrt(Dh/2Rc)). At Re = 998, the Dean number for Geometry 1 is approximately 631, compared to 499 for Geometry 2, consistent with the observed differences in secondary flow intensity.

### 4.5 Summary of Performance Comparison

As a whole the results show that Geometry 2 has the most favourable performance in terms of the pressure drop, with a pressure drop in the forward flow of about 1100 Pa at 1.5 m/s (Re ~ 3000) and a pressure drop in the reverse flow of about 3200 Pa at 1.5 m/s (Re ~ 3000), while also having minimal velocity loss. Geometry 1 is highly effective at inducing reverse flow resistance, as seen by a high pressure drop of ~6500 Pa, and considerable vorticity and velocity reduction. The importance of geometric design in achieving this balance between efficient forward flow and effective suppression of reverse flow is clearly indicated by these numerical results in passive flow rectification systems.

## 5. Conclusions

The present CFD study has demonstrated Tesla valves are very sensitive to geometry and Reynolds number. The following key conclusions are drawn from this investigation:

1. Geometry 2 (smooth-loop, Rc/Dh = 2.0, theta = 30 deg) is the optimal geometry in terms of hydraulic efficiency because it has the lowest forward pressure drop (~1100 Pa at Re ~ 3000) with a moderate reverse pressure drop (~3200 Pa), yielding a diodicity of approximately 2.91.

2. Geometry 1 (tight-loop, Rc/Dh = 1.25, theta = 45 deg) achieves the highest diodicity (~3.71 at Re ~ 3000) due to high vorticity, strong Dean-type secondary flows, and intense flow separation causing a high reverse pressure drop (~6500 Pa).

3. Diodicity increases monotonically with Reynolds number for both geometries, with more pronounced gains in the transitional regime (Re > 1000) where inertial effects dominate flow separation behaviour.

4. The curvature radius and branching angle are identified as the most influential geometric parameters: tighter curvature promotes vortex formation and higher diodicity, while smoother curvature reduces forward-flow losses.

5. The results show that the smooth turns and looping flow lead to low forward pressure losses, while the tight turns and sharp loops contribute to high reverse flow losses. Therefore, the design of an effective Tesla valve is a trade-off between diodicity and forward-flow pressure loss.

The study provides guidance for the design of efficient unpowered flow rectifiers. Future work will extend this investigation to unsteady pulsating flow conditions, compressible gas flows, and multiphase (liquid-vapour) flow scenarios that are increasingly relevant in thermal management and microfluidic applications.

## References

1. Park, H., & Kim, S. Y. (2026). Pressure drop characteristics of Tesla valve in fully turbulent flow. J. Fluids Eng., 148(3).

2. Tesla, N. (1920). Valvular conduit (U.S. Patent No. 1,329,559).

3. Han, Q., Liu, Z., Zhang, C., & Li, W. (2023). Int. J. Heat Mass Transfer, 214.

4. Forster, F. K., et al. (1995). ASME IMECE.

5. Truong, T. Q., & Nguyen, N. T. (2003). Nanotech 2003, 178-181.

6. Zhang, S., Winoto, S. H., & Low, H. T. (2007). 1st Int. Conf. Integration Micro Nanosystems, 15-19.

7. Gamboa, A. R., Morris, C. J., & Forster, F. K. (2005). J. Fluids Eng., 127(2), 339-346.

8. Mohammadzadeh, K., et al. (2013). J. Mechanics, 29(3), 527-534.

9. Nobakht, A. Y., Shahsavan, M., & Paykani, A. (2013). J. Applied Res. Tech., 11(6), 876-885.

10. Thompson, S. M., et al. (2014). J. Fluids Eng., 136(8).

11. Jin, Z. J., et al. (2018). Int. J. Hydrogen Energy, 43(18), 8888-8896.

12. Nguyen, Q. M., et al. (2021). Nature Communications, 12(1).

13. Thompson, S. M., et al. (2013). ASME IMECE.

14. Yontar, A. A., et al. (2021). J. Scientific Reports-A, (047), 47-67.

15. Qian, J. Y., et al. (2019). Int. J. Hydrogen Energy, 44(26), 13666-13674.

16. Monika, K., et al. (2021). Int. J. Heat Mass Transfer, 177.

17. Lu, Y. B., et al. (2022). Applied Thermal Engineering, 212.

18. Bohm, S., et al. (2022). Microsystems and Nanoengineering, 8(1).

19. Purwidyantri, A., & Nguyen, T. A. D. (2023). Chemosensors, 11(4).

20. Shi, Y., et al. (2026). Int. J. Heat Mass Transfer. Under review.

21. Han, J., et al. (2026). Applied Thermal Engineering. Under review.

22. Li, W., et al. (2023). Nature Communications, 14.

23. Qin, Z., & Wang, B. (2025). Int. J. Heat Mass Transfer, 239.

24. Li, W., et al. (2022). Int. J. Heat Mass Transfer, 187.

25. Qian, C., et al. (2025). Fluids, 10(10).

26. Bardell, R. L. (2000). PhD thesis, University of Washington.

27. Truong, T. V., & Nguyen, N. T. (2004). Sensors and Actuators A, 110(1-3), 126-132.

28. Gamboa, A. R., Morris, C. J., & Forster, F. K. (2005). J. Fluids Eng., 127(2), 339-346.

29. Razavi, S. E., & Shirani, E. (2018). Chem. Eng. Res. Design, 132, 101-112.

30. de Vries, S. F., et al. (2017). Int. J. Heat Mass Transfer, 105, 1-11.

31. Thompson, S. M., Ma, H. B., & Wilson, C. (2011). Exp. Thermal Fluid Sci., 35(7), 1265-1273.

32. Yang, K. S., Wang, C. C., & Tsai, P. H. (2019). Applied Thermal Engineering, 148, 963-972.

33. Kumar, R., Kumar, A., Goel, V., & Bhatt, G. (2022). Thermohydraulic performance of flat-tube radiator with twisted tape inserts. Applied Thermal Engineering, 212, 119281. https://doi.org/10.1016/j.applthermaleng.2022.119281

34. Kumar, A., & Layek, A. (2023). Thermohydraulic performance of solar air heater having twisted tape turbulators with multiple twists. Solar Energy, 257, 104-118. https://doi.org/10.1016/j.solener.2023.04.004

35. Yadav, S., & Prasad, R. K. (2025). Exergetic performance of solar air heater with internally corrugated absorber. Applied Thermal Engineering, 261, 126769. https://doi.org/10.1016/j.applthermaleng.2025.126769

Note: All text highlighted in yellow indicates new or revised content added in response to reviewer comments.