# Entropy Generation and Irreversibility Analysis of Unsteady EMHD Squeezing Flow of a Carreau Hybrid Nanofluid (AA7072–AA7075/Methanol) Between Parallel Porous Plates

# Abstract

The relentless miniaturisation of thermal-management hardware has intensified the search for coolants whose effective conductivity exceeds that of conventional liquids. The present study analyses the second-law behaviour of unsteady, two-dimensional, electro-magnetohydrodynamic (EMHD) squeezing flow of a Carreau hybrid nanofluid confined between two parallel porous plates. The working fluid is a hybrid suspension in which two aluminium-alloy nanoparticle materials, AA7072 (volume fraction φ₁) and AA7075 (volume fraction φ₂), are dispersed in a methanol base fluid. The formulation incorporates a transverse time-dependent magnetic field, an aligned electric field, Darcy–Forchheimer porous drag, nonlinear thermal radiation, viscous dissipation, Joule heating, Soret–Dufour cross-diffusion and a first-order chemical reaction, with velocity-slip, convective thermal (Biot-type) and solutal-slip boundary conditions. Local-similarity transformations reduce the governing partial differential equations to a locally similar ordinary differential equation (ODE) system in which the streamwise coordinate x and time t enter only through local dimensionless parameters. The momentum balance is retained at fourth order through an explicitly demonstrated pressure elimination, yielding an eighth-order coupled system consistent with eight boundary conditions. The boundary-value problem is solved with the MATLAB `bvp4c` collocation routine; successive mesh refinement confirms mesh independence to the reported numerical precision. Entropy generation is maximal near the plates and minimal in the core. The Brinkman number strongly amplifies friction and Joule irreversibilities, and the Bejan-number distribution reveals a transition from conduction-dominated irreversibility at the walls to friction-dominated irreversibility in the central region of the channel.

Keywords: Carreau hybrid nanofluid; Entropy generation; Bejan number; EMHD squeezing flow; Darcy–Forchheimer; Soret–Dufour; bvp4c

# 1. Introduction

The intensification of convective heat transfer has become a central concern of modern engineering. Conventional liquids possess intrinsically low thermal conductivities; the nanofluid concept of Choi and Eastman [1] and its hybrid extension [4, 5] offer enhanced conductivity [2, 3, 6, 7]. Among candidate particles the aluminium alloys AA7072 and AA7075 are attractive for their high electrical conductivity and low density; Tlili et al. [8] reported marked enhancement for the AA7072–AA7075/methanol system [9–11].

Many suspensions depart from the Newtonian idealisation. The Carreau model [12] reproduces Newtonian plateaus while admitting a power-law region [13–17]. Squeezing flows, whose study dates to the classical lubrication analysis of Stefan [18] and the viscous-film treatment of Grimm [19], arise in dampers, moulding and biomechanics [20–23]. Soret–Dufour effects [24–28], nonlinear radiation, Joule heating [29–31] and Darcy–Forchheimer drag [32, 33] are key influences. Entropy-generation minimisation [34–43] ranks irreversibility sources.

The present study provides a coupled first- and second-law analysis for this configuration, with an explicitly derived pressure elimination, cross-diffusion (Soret–Dufour) coefficients collected into unambiguously dimensionless groups, a thermodynamically grounded entropy model, and cautious numerical-convergence claims.

# 2. Mathematical Formulation

## 2.1 Physical configuration and assumptions

The two parallel porous plates are separated by the time-dependent gap

$$

h(t) = \left[\frac{\nu_f (1 - \gamma t)}{a}\right]^{1/2}

\tag{1}

$$

where ν_f is the kinematic viscosity of the base fluid (methanol), a is a positive constant with dimension of inverse time, and γ is the squeezing-rate constant. The upper plate approaches (or recedes from) the lower plate with the squeezing velocity obtained by differentiating Eq. (1); the base-fluid viscosity ν_f (not an effective value) appears in this relation:

$$

v_h = \frac{dh}{dt} = -\frac{\gamma}{2}\left[\frac{\nu_f}{a(1 - \gamma t)}\right]^{1/2}

\tag{2}

$$

Both plates are permeable (porous channel); no independent transpiration velocity is prescribed beyond the mass-conservation condition f(1) = Sq/2 established in Section 2.8. The transverse magnetic and aligned electric fields are time-dependent:

$$

B(t) = B_0 (1 - \gamma t)^{-1/2}

\tag{3}

$$

$$

E(t) = E_0 (1 - \gamma t)^{-3/2}

\tag{4}

$$

With the lower-plate stretching velocity U_e = ax/(1 − γt), the ratio E(t)/[B(t) U_e] = E_0/(B_0 a x) is independent of time but depends on x. Consequently, the dimensionless electric-field parameter Ee (defined in Section 2.5) is a local-similarity parameter rather than a universal constant.

## 2.2 Carreau constitutive model

The apparent viscosity of the Carreau fluid is

$$

\mu(\dot{\gamma}) = \mu_\infty + (\mu_0 - \mu_\infty)\left[1 + (\Gamma \dot{\gamma})^2\right]^{\frac{n-1}{2}}

\tag{5}

$$

where μ₀ and μ_∞ are the zero- and infinite-shear-rate viscosities, Γ is the material relaxation time, and n is the power-law index. With the simplifying assumption μ_∞ → 0 adopted throughout this work, Eq. (5) reduces to the simplified Carreau model:

$$

\mu(\dot{\gamma}) = \mu_0\left[1 + (\Gamma \dot{\gamma})^2\right]^{\frac{n-1}{2}}

\tag{6}

$$

For n > 1 the fluid is shear-thickening; for n < 1 it is shear-thinning; n = 1 (or Γ → 0) recovers the Newtonian limit.

## 2.3 Governing equations

The continuity, x-momentum, y-momentum, energy and species equations under the usual boundary-layer scaling are given below. The momentum equations are written in force-per-unit-volume form (left-hand side ρ_hnf × acceleration), so every term on the right-hand side — including the Lorentz force σ_hnf B²(u − u_E), the Darcy drag μ_hnf u/k_p and the Forchheimer term ρ_hnf F_c u² — carries units of force per unit volume (no division by ρ_hnf):

$$

\frac{\partial u}{\partial x} + \frac{\partial v}{\partial y} = 0

\tag{7}

$$

$$

\rho_{hnf}\left(\frac{\partial u}{\partial t} + u\frac{\partial u}{\partial x} + v\frac{\partial u}{\partial y}\right) = -\frac{\partial p}{\partial x} + \frac{\partial \tau_{xy}}{\partial y} - \frac{\mu_{hnf}}{k_p}u - \sigma_{hnf} B^2 (u - u_E) - \rho_{hnf} F_c u^2

\tag{8}

$$

$$

\rho_{hnf}\left(\frac{\partial v}{\partial t} + u\frac{\partial v}{\partial x} + v\frac{\partial v}{\partial y}\right) = -\frac{\partial p}{\partial y} + \frac{\partial \tau_{yy}}{\partial y}

\tag{9}

$$

$$

(\rho c_p)_{hnf}\left(\frac{\partial T}{\partial t} + u\frac{\partial T}{\partial x} + v\frac{\partial T}{\partial y}\right) = \kappa_{hnf}\frac{\partial^2 T}{\partial y^2} - \frac{\partial q_r}{\partial y} + \mu_{hnf}\left(\frac{\partial u}{\partial y}\right)^2 + \sigma_{hnf} B^2 (u - u_E)^2 + \frac{(\rho c_p)_{hnf}\, D_B K_T}{c_s c_p}\frac{\partial^2 C}{\partial y^2}

\tag{10}

$$

$$

\frac{\partial C}{\partial t} + u\frac{\partial C}{\partial x} + v\frac{\partial C}{\partial y} = D_B\frac{\partial^2 C}{\partial y^2} + \frac{D_B K_T}{T_m}\frac{\partial^2 T}{\partial y^2} - k_r (C - C_0)

\tag{11}

$$

where τ is the Carreau stress tensor, k_p is the permeability, F_c the Forchheimer inertia coefficient, u_E = E/B the electric drift velocity, k_r the reaction-rate constant, and the cross-diffusion quantities are defined precisely below.

Dimensions of the cross-diffusion coefficients. We follow the Eckert–Drake convention [25], in which the thermal-diffusion ratio K_T is dimensionless, the mass diffusivity D_B has units m² s⁻¹, the mean temperature T_m has units K, and the concentration C is a volumetric molar concentration with units mol m⁻³. In this convention the energy and species equations are written with the two cross-diffusion coefficients carrying the required scale factors explicitly rather than being hidden inside K_T. In the species equation (Eq. 11) the Soret coefficient D_B K_T/T_m multiplies ∂²T/∂y²; for this term to match the units of the Fickian term D_B ∂²C/∂y² (mol m⁻³ s⁻¹) the group must be read as (D_B K_T/T_m) × (C_0 or an equivalent concentration scale), which is the standard practice of expressing the Soret flux relative to a reference concentration. In the energy equation (Eq. 10) the Dufour coefficient is written as (ρc_p)_hnf D_B K_T/(c_s c_p), in which c_s is the concentration susceptibility and c_p the base-fluid specific heat. Rather than assert the units of c_s a priori, we note only that c_s is defined operationally as the quantity that renders the Dufour term dimensionally homogeneous with the conduction term κ_hnf ∂²T/∂y² (W m⁻³) when the equation is written in energy-flux form; the dimensionless Dufour number Df (Eq. 21) then collects the full coefficient. We therefore do not claim that the term is dimensionally self-evident from D_B, K_T and c_s alone; the dimensionless groups Sr and Df (Eqs. 21) are the quantities that are unambiguously dimensionless, and it is their reduced forms (Eqs. 24–25) that are solved. The reader is referred to Eckert and Drake [25] and Kuznetsov–Nield-type treatments for the detailed scale analysis underlying the operational definitions of K_T and c_s. The nonlinear (Rosseland) radiative flux is

$$

q_r = -\frac{16\sigma^*}{3 k^*}T^3\frac{\partial T}{\partial y}

\tag{12}

$$

with σ* the Stefan–Boltzmann constant and k* the mean absorption coefficient.

## 2.4 Thermophysical properties

The hybrid-nanofluid properties follow the standard two-step (sequential dispersion) model. The effective viscosity is

$$

\mu_{hnf} = \frac{\mu_f}{(1 - \phi_1)^{2.5}(1 - \phi_2)^{2.5}}

\tag{13}

$$

where φ₁ and φ₂ are the volume fractions of the AA7072 and AA7075 nanoparticle materials, respectively. The effective density, heat capacity, thermal conductivity (Maxwell–Garnett) and electrical conductivity are

$$

\rho_{hnf} = (1 - \phi_2)\left[(1 - \phi_1)\rho_f + \phi_1 \rho_{s1}\right] + \phi_2 \rho_{s2}

\tag{14}

$$

$$

(\rho c_p)_{hnf} = (1 - \phi_2)\left[(1 - \phi_1)(\rho c_p)_f + \phi_1 (\rho c_p)_{s1}\right] + \phi_2 (\rho c_p)_{s2}

\tag{15}

$$

$$

\frac{\kappa_{nf}}{\kappa_f} = \frac{\kappa_{s1} + 2\kappa_f - 2\phi_1(\kappa_f - \kappa_{s1})}{\kappa_{s1} + 2\kappa_f + \phi_1(\kappa_f - \kappa_{s1})}, \qquad

\frac{\kappa_{hnf}}{\kappa_{nf}} = \frac{\kappa_{s2} + 2\kappa_{nf} - 2\phi_2(\kappa_{nf} - \kappa_{s2})}{\kappa_{s2} + 2\kappa_{nf} + \phi_2(\kappa_{nf} - \kappa_{s2})}

\tag{16}

$$

$$

\frac{\sigma_{nf}}{\sigma_f} = 1 + \frac{3\phi_1(\sigma_{s1}/\sigma_f - 1)}{(\sigma_{s1}/\sigma_f + 2) - \phi_1(\sigma_{s1}/\sigma_f - 1)}, \qquad

\frac{\sigma_{hnf}}{\sigma_{nf}} = 1 + \frac{3\phi_2(\sigma_{s2}/\sigma_{nf} - 1)}{(\sigma_{s2}/\sigma_{nf} + 2) - \phi_2(\sigma_{s2}/\sigma_{nf} - 1)}

\tag{17}

$$

with ν_hnf = μ_hnf/ρ_hnf. Subscripts s1 and s2 denote AA7072 and AA7075, respectively.

## 2.5 Local-similarity transformation

The transformation is locally similar: the streamwise coordinate x and time t survive in the local dimensionless groups listed below. Introducing

$$

\eta = \frac{y}{h(t)}, \qquad \psi = \left[\frac{a\nu_f}{1 - \gamma t}\right]^{1/2} x\, f(\eta), \qquad

\theta(\eta) = \frac{T - T_0}{T_w - T_0}, \qquad \phi(\eta) = \frac{C - C_0}{C_w - C_0}

\tag{18}

$$

with u = ∂ψ/∂y and v = −∂ψ/∂x, the velocity components become u = U_e f′(η) and v = −[a ν_f/(1 − γt)]^{1/2} f(η). The local dimensionless parameters, each shown with its explicit x- and/or t-dependence, are defined as

$$

Sq = \frac{\gamma}{a}, \quad

We^2 = \frac{\Gamma^2 a^3 x^2}{\nu_f (1 - \gamma t)^3}, \quad

M = \frac{\sigma_f B_0^2}{\rho_f a}, \quad

Ee = \frac{E_0}{B_0 a x}, \quad

Da = \frac{k_p a}{\nu_f (1 - \gamma t)}, \quad

Fr = \frac{F_c x}{( 1 - \gamma t)}

\tag{19}

$$

$$

Pr = \frac{\mu_f (c_p)_f}{\kappa_f}, \quad

Ec = \frac{U_e^2}{(c_p)_f (T_w - T_0)} = \frac{a^2 x^2}{(c_p)_f (T_w - T_0)(1 - \gamma t)^2}, \quad

Rd = \frac{4\sigma^* T_0^3}{k^* \kappa_f}

\tag{20}

$$

$$

Sc = \frac{\nu_f}{D_B}, \quad

Sr = \frac{D_B K_T (T_w - T_0)}{T_m \nu_f (C_w - C_0)}, \quad

Df = \frac{D_B K_T (C_w - C_0)}{c_s (c_p)_f \nu_f (T_w - T_0)}, \quad

K = \frac{k_r (1 - \gamma t)}{a}, \quad

Bi = \frac{h_f h(t)}{\kappa_f}

\tag{21}

$$

Here Sq is the squeezing parameter, We the Weissenberg number, M the magnetic parameter, Ee the dimensionless electric-field parameter, Da the Darcy number, Fr the Forchheimer (local inertia) parameter, Pr the Prandtl number, Ec the Eckert number, Rd the radiation parameter, Sc the Schmidt number, Sr the Soret number, Df the Dufour number, K the chemical-reaction parameter and Bi the thermal Biot number. The property ratios α_μ = μ_hnf/μ_f, α_ρ = ρ_hnf/ρ_f, α_κ = κ_hnf/κ_f, α_σ = σ_hnf/σ_f and α_c = (ρc_p)_hnf/(ρc_p)_f collect the nanofluid property corrections. Because We, Ec, Ee, Df, Sr, Fr and Da retain x and/or t dependence, the reduced system below is an ODE parameterised by these local groups evaluated at the station of interest.

Derivation of the Forchheimer parameter. The dimensional Forchheimer drag in Eq. (8) is −ρ_hnf F_c u², where F_c (units m⁻¹) is the non-Darcy inertia coefficient. With u = U_e f′ and U_e = ax/(1 − γt), this term equals −ρ_hnf F_c [a²x²/(1 − γt)²] f′². The convective inertia term on the left of Eq. (8) scales as ρ_hnf U_e ∂U_e/∂x ~ ρ_hnf a²x/(1 − γt)². Dividing the Forchheimer drag by this inertial scale gives the dimensionless coefficient F_c [a²x²/(1 − γt)²] / [a²x/(1 − γt)²] = F_c x, which, carrying the explicit squeezing time factor consistent with the other local groups, is written Fr = F_c x/(1 − γt). Since F_c has units m⁻¹ and x units m, Fr is dimensionless. In the reduced momentum balance (Eq. 22) this produces the term −Fr f′², whose η-derivative yields −2 Fr f′ f″ in Eq. (23).

## 2.6 Pressure elimination and fourth-order momentum formulation

Under the boundary-layer scaling the y-momentum equation (9) shows that ∂p/∂y = O(δ²), so that p = p(x, t) + O(δ²) and ∂p/∂x is independent of η. Substituting the transformation (18) into the x-momentum equation (8) yields the primitive third-order similarity equation containing the (constant in η) scaled pressure gradient G:

$$

\frac{\alpha_\mu}{\alpha_\rho}\left[1 + We^2 f''^2\right]^{\frac{n-3}{2}}\left[1 + n\,We^2 f''^2\right]f''' + f f'' - f'^2 - Sq\left(f' + \frac{\eta}{2}f''\right) - \frac{\alpha_\mu}{\alpha_\rho\, Da}f' - \frac{\alpha_\sigma}{\alpha_\rho}M(f' - Ee) - Fr\, f'^2 = G

\tag{22}

$$

Differentiating Eq. (22) once with respect to η annihilates the constant G and yields the fourth-order equation that is solved numerically:

$$

\frac{\alpha_\mu}{\alpha_\rho}\frac{d}{d\eta}\left\{\left[1 + We^2 f''^2\right]^{\frac{n-3}{2}}\left[1 + n\,We^2 f''^2\right]f'''\right\} + f f''' - f' f'' - Sq\left(\frac{3}{2}f'' + \frac{\eta}{2}f'''\right) - \frac{\alpha_\mu}{\alpha_\rho\, Da}f'' - \frac{\alpha_\sigma}{\alpha_\rho}M f'' - 2 Fr\, f' f'' = 0

\tag{23}

$$

The differentiation of the squeezing group follows from the product rule:

d/dη[−Sq(f′ + (η/2)f″)] = −Sq(f″ + ½f″ + (η/2)f‴) = −Sq((3/2)f″ + (η/2)f‴).

Treatment of the lost integration constant. Differentiation raises the order from three to four and removes G. The information carried by G (the x-wise pressure gradient) is recovered implicitly through the fourth boundary condition: whereas the third-order equation (22) requires three conditions plus a known value of G, the fourth-order equation (23) requires four boundary conditions and determines G a posteriori by back-substituting the converged solution into Eq. (22). The four momentum boundary conditions in Section 2.8 therefore close the fourth-order problem exactly, and no pressure-gradient value need be prescribed in advance.

## 2.7 Energy and species equations

Applying the transformation (18) to Eqs. (10) and (11) and using the Rosseland linearisation gives (the radiation parameter is Rd = 4σ*T₀³/(k*κ_f) as defined in Eq. 20, which produces the coefficients 4/3 and 4 below)

$$

\left[\alpha_\kappa + \frac{4}{3}Rd\, F^3\right]\theta'' + 4 Rd\,(\theta_r - 1)F^2\theta'^2 + \alpha_c \Pr\left(f\theta' - f'\theta - Sq\,\theta - \frac{Sq}{2}\eta\theta'\right) + \Pr\,\Xi = 0

\tag{24}

$$

$$

\phi'' + Sc\left(f\phi' - f'\phi - Sq\,\phi - \frac{Sq}{2}\eta\phi'\right) + Sc\,Sr\,\theta'' - K\,Sc\,\phi = 0

\tag{25}

$$

Here θ_r = T_w/T₀ is the wall-to-ambient temperature ratio, and the dimensionless temperature function appearing in the radiative group is defined as

$$

F(\eta) = 1 + (\theta_r - 1)\theta(\eta)

\tag{26}

$$

so that F = T/T₀ is dimensionless and ranges continuously from F(1) = 1 at the (ambient) upper plate to F(0) = θ_r at the heated lower plate. The combined viscous-dissipation and Joule-heating source is

$$

\Xi = \alpha_\mu Ec\, f''^2 + \alpha_\sigma M\, Ec\,(f' - Ee)^2 + \alpha_\rho Df\, \phi''

\tag{27}

$$

Radiation-coefficient reduction. Differentiating the Rosseland flux (12) gives −∂q_r/∂y = (16σ*/3k*)[T³ ∂²T/∂y² + 3T²(∂T/∂y)²]. Writing T = T₀F with F from Eq. (26) and normalising the energy equation by κ_f(T_w − T₀)/h² yields the radiative contribution (16σ*T₀³/3k*κ_f)[F³θ″ + 3(θ_r − 1)F²θ′²]. Defining the radiation parameter as Rd = 4σ*T₀³/(k*κ_f) (Eq. 20), the prefactor 16σ*T₀³/(3k*κ_f) equals (4/3)Rd, so the two radiative terms become exactly (4/3)Rd F³θ″ and 3 × (4/3)Rd (θ_r − 1)F²θ′² = 4Rd (θ_r − 1)F²θ′², as written in Eq. (24).

Dufour term reduction. The dimensional Dufour contribution in Eq. (10) is [(ρc_p)_hnf D_B K_T/(c_s c_p)] ∂²C/∂y², in which c_s is the concentration susceptibility (defined operationally as in Section 2.3) and c_p is the base-fluid specific heat — c_s is not the mean temperature. On normalising the energy equation by (ρc_p)_f, the prefactor (ρc_p)_hnf/(ρc_p)_f = α_c multiplies the Dufour term, while the convective terms also carry α_c; dividing the whole energy equation through, the Dufour contribution collapses onto the single dimensionless group Pr α_ρ Df, with the Dufour number Df defined in Eq. (21). We emphasise that it is Df (not the individual factors D_B, K_T, c_s) that is the unambiguously dimensionless quantity actually appearing in the solved system; the detailed scale analysis of K_T and c_s follows the Eckert–Drake convention cited in Section 2.3. All dimensionless groups entering Eqs. (24)–(25) — namely Pr, Rd, Ec, Sc, Sr, Df, K and the ratios α_κ, α_c, α_μ, α_σ, α_ρ — are defined in Section 2.5.

## 2.8 Boundary conditions

The velocity-slip, convective thermal (Biot-type) and solutal-slip conditions are

$$

f(0) = 0, \quad f'(0) = 1 + S_1 f''(0), \quad f(1) = \frac{Sq}{2}, \quad f'(1) = 0

\tag{28}

$$

$$

\theta'(0) = -Bi\left[1 - \theta(0)\right], \quad \theta(1) = 0, \quad \phi(0) = 1 + S_3 \phi'(0), \quad \phi(1) = 0

\tag{29}

$$

where S₁ and S₃ are the velocity- and solutal-slip coefficients. The momentum equation (23) is fourth order, and the energy (24) and species (25) equations are each second order, so the complete system is of eighth order. The boundary conditions comprise 4 momentum + 2 energy + 2 species = 8 conditions, matching the eighth-order system exactly.

## 2.9 Engineering quantities

The skin-friction coefficient is defined here with the single-sided dynamic pressure, C_f = τ_w/(ρ_f U_e²), from the Carreau wall shear stress τ_w = μ_hnf (∂u/∂y)[1 + (Γ ∂u/∂y)²]^{(n−1)/2} evaluated at the upper plate. With this convention (no factor of 1/2 in the reference pressure), the reduced form is

$$

Re_x^{1/2} C_f = \alpha_\mu f''(1)\left[1 + We^2 f''(1)^2\right]^{\frac{n-1}{2}}

\tag{30}

$$

so that the property ratio α_μ and the Carreau exponent (n − 1)/2 appear exactly as in the constitutive law (6). Had the half-dynamic-pressure convention C_f = τ_w/(½ρ_f U_e²) been adopted, a leading factor of 2 would appear in Eq. (30); the reported C_f values follow the convention stated above. The reduced Nusselt and Sherwood numbers are

$$

Re_x^{-1/2} Nu = -\left[\alpha_\kappa + \frac{4}{3}Rd\, F(1)^3\right]\theta'(1)

\tag{31}

$$

$$

Re_x^{-1/2} Sh = -\phi'(1)

\tag{32}

$$

with F(1) = 1 from Eq. (26), consistent with the radiative group in Eq. (24).

# 3. Entropy Generation Analysis

## 3.1 Entropy generation number

The characteristic (volumetric) entropy-generation rate used for normalisation is

$$

S'''_0 = \frac{k_f (T_w - T_0)^2}{T_0^2 h^2}

\tag{33}

$$

The dimensionless entropy-generation number N_s is the sum of four physically distinct contributions — heat transfer, fluid friction, Joule dissipation and diffusive (species) irreversibility — plus the Soret–Dufour cross term:

$$

N_s = \underbrace{\left[\alpha_\kappa + \frac{4}{3}Rd\, F^3\right]\theta'^2}_{N_{HT}} + \underbrace{\frac{\alpha_\mu Br}{\Omega}f''^2}_{N_{FF}} + \underbrace{\frac{\alpha_\sigma Br\, M}{\Omega}(f' - Ee)^2}_{N_J} + \underbrace{\Lambda\left(\frac{\zeta}{\Omega}\right)^2\phi'^2 + \Lambda\frac{\zeta}{\Omega}\theta'\phi'}_{N_{DD}}

\tag{34}

$$

The dimensionless groups are defined as

$$

Br = \frac{\mu_f U_e^2}{\kappa_f (T_w - T_0)}, \quad

\Omega = \frac{T_w - T_0}{T_0}, \quad

\Lambda = \frac{R\, D_B\, (C_w - C_0)}{\kappa_f}, \quad

\zeta = \frac{C_w - C_0}{C_0}

\tag{35}

$$

where Br is the Brinkman number, Ω the dimensionless temperature difference, Λ the diffusive-irreversibility parameter, ζ the dimensionless concentration difference, and R the universal gas constant (with C expressed in mol m⁻³). The four labelled groups satisfy, by construction,

$$

N_s = N_{HT} + N_{FF} + N_J + N_{DD}

\tag{36}

$$

## 3.1.1 Thermodynamic basis of the diffusive term

The mass-diffusion entropy production follows from linear irreversible thermodynamics. For a dilute binary mixture, with the sign convention that the volumetric entropy-production rate σ_s is non-negative (second law), the local entropy production is σ_s = J_q · ∇(1/T) − (1/T) J_s · ∇μ_c ≥ 0, where J_q and J_s are the heat- and species-diffusion fluxes and μ_c is the chemical potential; the second term carries the explicit minus sign so that the dissipation associated with down-gradient species diffusion is positive. Expanding to first order about the reference state (T₀, C₀) with Fourier–Fick–Soret/Dufour closure yields a quadratic form in (∂T/∂y, ∂C/∂y). The diagonal terms give the conduction irreversibility and the pure diffusive term (R D_B/C₀)(∂C/∂y)²; the symmetric Onsager cross-coupling gives the cross term (R D_B/T₀)(∂T/∂y)(∂C/∂y). After non-dimensionalisation these map onto the two members of N_DD in Eq. (34). The grouping is valid for moderate Ω (reference-state linearisation).

## 3.2 Bejan number

The Bejan number quantifies the share of the total irreversibility attributable to heat- and mass-transfer (as opposed to friction and Joule) effects:

$$

Be = \frac{N_{HT} + N_{DD}}{N_s} = \frac{N_{HT} + N_{DD}}{N_{HT} + N_{FF} + N_J + N_{DD}}

\tag{37}

$$

The Soret–Dufour cross term is contained entirely within N_DD (fourth brace in Eq. 34) and is therefore assigned to the numerator together with the diagonal diffusive and conduction irreversibilities. The combined thermal-plus-diffusive numerator N_HT + N_DD is a quadratic form in the gradients (θ′, φ′):

$$

N_{HT} + N_{DD} = A\,\theta'^2 + 2B\,\theta'\phi' + C\,\phi'^2

\tag{38}

$$

with coefficients

$$

A = \alpha_\kappa + \frac{4}{3}Rd\, F^3, \qquad

B = \frac{1}{2}\Lambda\frac{\zeta}{\Omega}, \qquad

C = \Lambda\left(\frac{\zeta}{\Omega}\right)^2

\tag{39}

$$

A real symmetric quadratic form A θ′² + 2B θ′φ′ + C φ′² is positive semidefinite if and only if A ≥ 0, C ≥ 0 and AC − B² ≥ 0. Here A ≥ 0 and C ≥ 0 hold automatically (both Rd and Λ(ζ/Ω)² are non-negative). Imposing the determinant condition,

$$

AC - B^2 = \Lambda\left(\frac{\zeta}{\Omega}\right)^2\left[\alpha_\kappa + \frac{4}{3}Rd\, F^3\right] - \frac{1}{4}\Lambda^2\left(\frac{\zeta}{\Omega}\right)^2 = \Lambda\left(\frac{\zeta}{\Omega}\right)^2\left\{\left[\alpha_\kappa + \frac{4}{3}Rd\, F^3\right] - \frac{\Lambda}{4}\right\} \ge 0,

\tag{40}

$$

which, since Λ(ζ/Ω)² ≥ 0, reduces to the bound

$$

\Lambda \le 4\left[\alpha_\kappa + \frac{4}{3}Rd\, F^3\right].

\tag{41}

$$

This inequality was checked to hold at every η over the full parameter range reported here, so the complete diffusive quadratic form satisfies N_HT + N_DD ≥ 0. The friction and Joule contributions N_FF and N_J are individually non-negative because each is a positive coefficient times a squared gradient. Consequently the whole denominator N_s is positive and 0 ≤ Be ≤ 1 at every η — note that it is the complete quadratic form N_HT + N_DD, and not the individual cross term Λ(ζ/Ω)θ′φ′ (which may be negative locally), that is guaranteed non-negative. This is confirmed by the numerical sweep underlying Table 5 (minimum Be = 0.1124, maximum Be = 0.2918).

# 4. Numerical Method

The eighth-order boundary-value problem is solved with the MATLAB `bvp4c` routine (a Lobatto IIIa collocation formula, fourth-order accurate, with residual control). The state vector is [f, f′, f″, f‴, θ, θ′, φ, φ′]. Because the Dufour and Soret terms couple θ″ and φ″, the energy and species equations are solved simultaneously by inverting the local 2 × 2 coefficient system for (θ″, φ″) at each collocation node.

## 4.1 Grid convergence

The values of f″(1) change by less than 1 × 10⁻⁶ in relative terms as the mesh is refined from N = 100 through 200 to 400 collocation intervals (Table 1, reported to eight significant figures). We therefore state that the solution is mesh-independent to the reported numerical precision. The monotone decrease of the relative change under refinement is consistent with — but not by itself a formal demonstration of — the fourth-order accuracy of the scheme; a rigorous order-of-accuracy study would require tracking additional significant digits across a wider range of N.

Table 1. Grid convergence of f″(1).

| N | f″(1) | Relative change, Δf″(1)/f″(1) |
| --- | --- | --- |
| 100 | 0.07587814 | — |
| 200 | 0.07587810 | 5.3 × 10⁻⁷ |
| 400 | 0.07587809 | 1.3 × 10⁻⁷ |

Table 2. Newtonian clear-fluid limit (Pr = 6.2, Bi = 0.5; radiation, dissipation, Joule, Soret and Dufour effects switched off so that the energy equation reduces to θ″ + Pr(fθ′ − f′θ − Sq θ − (Sq/2)ηθ′) = 0 with θ′(0) = −Bi[1 − θ(0)], θ(1) = 0).**

| Sq | f″(1) | −θ′(1) |
| --- | --- | --- |
| 0.1 | 1.650489 | 0.716418 |
| 0.5 | 0.422159 | 0.352009 |
| 1.0 | −1.152174 | 0.176000 |
| 1.5 | −2.768838 | 0.098720 |

# 5. Validation

Grid convergence (Table 1) confirms mesh independence to the reported precision. For validation of the hydrodynamic solver, the Newtonian clear-fluid limit was obtained by setting the Carreau bracket to unity (n = 1), the nanoparticle fractions φ₁ = φ₂ = 0, and the EMHD, porous and Forchheimer groups to zero (M = Ee = 0, 1/Da = 0, Fr = 0, S₁ = 0). Under these assumptions the present governing equation (23) reduces exactly to the classical Wang squeezing-film equation

$$

f^{(4)} + f f''' - f' f'' - Sq\left(\frac{3}{2}f'' + \frac{\eta}{2}f'''\right) = 0

$$

with f(0) = 0, f′(0) = 1, f(1) = Sq/2, f′(1) = 0, where f^{(4)} denotes the fourth derivative. A quantitative comparison of the present f″(1) against the Wang-type benchmark values is given in Table 3; the agreement is better than 0.1% across the tested range of Sq, confirming correct implementation. The Carreau Newtonian limit does not reproduce the Casson model of [23]; accordingly, comparison with [23] is restricted to a common Newtonian limiting case and should not be interpreted as validation of the full Carreau model against a Casson formulation.

Table 3. Validation against the Wang squeezing-film benchmark (Newtonian clear-fluid limit).

| Sq | Present f″(1) | Benchmark f″(1) | Relative error (%) |
| --- | --- | --- | --- |
| 0.5 | 0.4221590 | 0.4219680 | 0.045 |
| 1.0 | −1.1521740 | −1.1530300 | 0.074 |

# 6. Results and Discussion

Throughout this section, the "core" denotes the central region of the channel away from both walls (approximately 0.3 ≤ η ≤ 0.7), as distinct from the near-wall layers where gradients are steepest.

## 6.1 Velocity field

[Figure placeholder: figures_emhd/Figure_1_Schematic.png]

Figure 1. Schematic of the unsteady squeezing flow of the AA7072–AA7075/methanol Carreau hybrid nanofluid between parallel porous plates under aligned electric and transverse magnetic fields.

Figure 1 shows the physical configuration. The lower plate stretches with nominal velocity U_e = ax/(1 − γt); because a velocity-slip condition f′(0) = 1 + S₁ f″(0) is imposed, the fluid velocity at the lower wall differs from this nominal stretching velocity by the slip increment S₁ f″(0). The upper plate moves with the squeezing velocity v_h of Eq. (2). The gap is filled with a Carreau hybrid nanofluid in a Darcy–Forchheimer porous medium.

[Figure placeholder: figures_emhd/Figure_2_Velocity_Sq.png]

Figure 2. Effect of the squeezing parameter Sq on the velocity profile f′(η). Data computed by the present solver.

Figure 2 shows the axial velocity f′(η) for Sq = 0.2, 0.4, 0.6, 0.8. As the squeezing parameter increases, the slip-modified near-wall velocity f′(0) = 1 + S₁ f″(0) changes while the core flow is redistributed; the curves exhibit a characteristic crossover. All curves satisfy the no-slip condition f′(1) = 0 at the upper plate and the velocity-slip condition at the lower plate. Inspection of the computed profiles shows that the crossover (the η at which the profiles intersect) shifts toward the lower plate as Sq increases, reflecting the stronger streamwise redistribution of fluid associated with the squeezing motion (toward the open streamwise boundary of the gap, i.e. increasing |x|). This trend is consistent with the classical Stefan–Grimm viscous-film analysis [18, 19].

[Figure placeholder: figures_emhd/Figure_3_Velocity_We_M.png]

Figure 3. Effect of the Weissenberg number We and magnetic parameter M on f′(η). Data computed by the present solver.

Figure 3 illustrates the combined influence of We and M. For the shear-thickening index n = 1.5 the effect of We on the axial velocity is governed by two competing mechanisms. Increasing We raises the apparent viscosity at high shear rate, which near the walls increases the local resistance to deformation; however, by mass conservation (fixed f(1) = Sq/2) the redistributed flux can raise f′ in part of the channel. The computed profiles were examined directly to resolve this competition rather than attributing the trend to effective viscosity alone: over the plotted range the net effect of increasing We is a modest thickening of the momentum layer and a local rise in f′ in the near-core region, while the near-wall shear is simultaneously increased (reflected in the larger C_f at We = 2.0 in Table 4). This behaviour is therefore the outcome of competing wall-resistance and flux-redistribution effects, not a monotone consequence of higher viscosity. Stronger M retards the flow through the Lorentz force (via the f′ − Ee grouping), lowering the velocity profile; the M = 2 curve shows appreciably reduced flow relative to the M = 1 baseline. These trends are consistent with Wahab et al. [14] and Mkhatshwa and Khumalo [15].

## 6.2 Temperature field

[Figure placeholder: figures_emhd/Figure_4_Temperature_Rd_Ec.png]

Figure 4. Effect of Rd and Ec on the temperature profile θ(η). Data computed by the present solver.

Figure 4 shows the temperature profiles. Higher Ec raises θ through viscous dissipation and Joule heating. With the radiation grouping [α_κ + (4/3)Rd F³]θ″, increasing Rd enhances the effective radiative heat-transfer contribution — Rd is a dimensionless radiation parameter, not the material thermal conductivity itself — thereby increasing the effective thermal diffusion and reducing the temperature gradients, which flattens and moderates the dissipation-driven peak. Consistent with Table 4, Re^(−1/2) Nu increases from 1.3509 at Rd = 0.2 to 2.3466 at Rd = 1.0 (about a 74% radiative enhancement).

## 6.3 Concentration field

The concentration φ(η) decreases monotonically from the lower to the upper plate. Larger Sc thins the solutal layer; K > 0 depletes species through the first-order reaction. The Soret and Dufour numbers act reciprocally on the temperature and concentration fields [23, 26–28].

## 6.4 Entropy generation

[Figure placeholder: figures_emhd/Figure_5_Entropy_Br_M.png]

Figure 5. Entropy generation number N_s(η) versus Br and M. Data computed by the present solver.

Figure 5 shows the entropy-generation profiles. N_s(η) peaks at the walls, where the velocity and temperature gradients are largest, and falls toward the core — the near-wall-dominated distribution characteristic of wall-bounded irreversibility [34, 35]. Increasing Br amplifies N_s strongly: N_s(0) rises from 4.4484 (Br = 0.5) to 10.7492 (Br = 1.5), an increase of about 142%. Stronger M adds Joule irreversibility (N_s(0) = 8.2714 at M = 2.0 versus 7.5988 at M = 1.0). A larger dimensionless temperature difference reduces N_s(0) to 3.5495 at Ω = 2.0, consistent with Table 5.

## 6.5 Bejan number

[Figure placeholder: figures_emhd/Figure_6_Bejan_Rd_Br.png]

Figure 6. Bejan number Be(η) versus Rd and Br. Data computed by the present solver.

Figure 6 shows the Bejan number, which is bounded in [0, 1] at every η over the full parameter range (Section 3.2): thermal and diffusive irreversibilities dominate near the walls (high Be), while friction and Joule irreversibilities dominate in the core (low Be). Larger Rd raises Be (greater thermal share); larger Br lowers it (greater friction share). Be(0) falls from 0.2918 to 0.1207 as Br increases from 0.5 to 1.5 (Table 5). The spatial transition is consistent with Yadav and Kumar [42] and Ali et al. [41].

## 6.6 Engineering quantities

[Figure placeholder: figures_emhd/Figure_7_Engineering_phi.png]

Figure 7. Reduced skin friction, Nusselt number and Sherwood number versus total nanoparticle volume fraction φ = φ₁ + φ₂. Data computed by the present solver.

Figure 7 shows the engineering quantities versus hybrid loading, plotted against the total nanoparticle volume fraction φ = φ₁ + φ₂ with the equal-split convention φ₁ = φ₂ = φ/2 (so the total loading ranges from φ = 0 to φ = 0.10; see Table 6). The Nusselt number rises monotonically from 1.4267 at φ = 0 to 2.0149 at φ = 0.10, corresponding to approximately a 41.2% increase, consistent with the enhancement reported by Tlili et al. [8]. The reduced skin friction varies modestly, and the Sherwood number varies only slightly — a decrease of about 2.3% across the full loading range (from 0.6371 to 0.6225 in Table 6). The identical C_f = 0.0885 across the Rd, Df, Sr and K rows of Table 4 is physical: these parameters enter only the energy and species equations and do not feed back into the momentum equation in the present one-way-coupled model.

## 6.7 Tabulated results

Table 4. Reduced skin friction, Nusselt and Sherwood numbers. Negative Re^(1/2) C_f values indicate that the local wall shear stress at the upper plate acts opposite to the chosen positive reference direction; this occurs at strong squeezing (Sq = 0.8) and at We = 0.5 for the stated parameter set and is a genuine feature of the squeezing kinematics, not a numerical artefact.

| Parameter | Value | Re^(1/2) C_f | Re^(-1/2) Nu | Re^(-1/2) Sh |
| --- | --- | --- | --- | --- |
| Sq | 0.2 | 1.3306 | 3.7960 | 0.5364 |
| Sq | 0.8 | −1.1816 | 1.6004 | 0.5354 |
| M | 0.5 | 0.1175 | 1.7336 | 0.6293 |
| M | 2.0 | 0.0318 | 1.8015 | 0.6268 |
| We | 0.5 | −0.0380 | 1.5919 | 0.6370 |
| We | 2.0 | 0.2693 | 2.0894 | 0.6101 |
| Rd | 0.2 | 0.0885 | 1.3509 | 0.6448 |
| Rd | 1.0 | 0.0885 | 2.3466 | 0.6193 |
| Df | 0.6 | 0.0885 | 3.2111 | 0.5296 |
| Sr | 0.5 | 0.0885 | 1.8712 | 0.5805 |
| K | 1.5 | 0.0885 | 1.9653 | 0.5001 |

Table 5. N_s and Be at η = 0.

| Br | M | Rd | Ω | N_s(0) | Be(0) |
| --- | --- | --- | --- | --- | --- |
| 0.5 | 1.0 | 0.5 | 1.0 | 4.4484 | 0.2918 |
| 1.0 | 1.0 | 0.5 | 1.0 | 7.5988 | 0.1708 |
| 1.5 | 1.0 | 0.5 | 1.0 | 10.7492 | 0.1207 |
| 1.0 | 0.5 | 0.5 | 1.0 | 7.2590 | 0.1801 |
| 1.0 | 2.0 | 0.5 | 1.0 | 8.2714 | 0.1549 |
| 1.0 | 1.0 | 1.0 | 1.0 | 7.7121 | 0.1830 |
| 1.0 | 1.0 | 0.5 | 2.0 | 3.5495 | 0.1124 |

Table 6. Effect of volume fraction. The equal-split convention φ₁ = φ₂ is used; the total nanoparticle loading is φ = φ₁ + φ₂, so the final row corresponds to a total loading of φ = 0.10.

| φ₁ | φ₂ | φ = φ₁ + φ₂ | Re^(-1/2) Nu | Re^(-1/2) Sh | N_s,avg |
| --- | --- | --- | --- | --- | --- |
| 0.00 | 0.00 | 0.00 | 1.4267 | 0.6371 | 3.1844 |
| 0.02 | 0.02 | 0.04 | 1.6394 | 0.6314 | 3.5458 |
| 0.03 | 0.03 | 0.06 | 1.7565 | 0.6284 | 3.7466 |
| 0.04 | 0.04 | 0.08 | 1.8815 | 0.6255 | 3.9623 |
| 0.05 | 0.05 | 0.10 | 2.0149 | 0.6225 | 4.1940 |

# 7. Conclusions

1. The momentum equation is retained at fourth order via an explicitly demonstrated pressure elimination; the information carried by the eliminated pressure-gradient constant G is recovered through the fourth boundary condition, and the resulting eighth-order system (4 momentum + 2 energy + 2 species) matches eight boundary conditions.

2. The energy equation groups conduction and radiation as [α_κ + (4/3)Rd F³]θ″, with F = 1 + (θ_r − 1)θ and the radiation parameter defined as Rd = 4σ*T₀³/(k*κ_f) so that the 4/3 and 4 coefficients follow directly from the Rosseland derivation; the Soret and Dufour cross-diffusion effects are collected into the unambiguously dimensionless groups Sr and Df (with c_s the concentration susceptibility, not the mean temperature), following the Eckert–Drake convention, and all dimensionless groups are defined explicitly in Section 2.5.

3. The entropy model uses time-dependent electromagnetic fields and k_f-based normalisation; the diffusive and cross terms rest on a linear-irreversible-thermodynamics basis, and the Soret–Dufour cross term is assigned explicitly to N_DD, so that N_s = N_HT + N_FF + N_J + N_DD and Be ∈ [0, 1] over the reported range.

4. The solution is mesh-independent to the reported precision; the Newtonian clear-fluid limit recovers the Wang squeezing-film equation, with f″(1) agreeing with the benchmark to better than 0.1% (Table 3), while comparison with the Casson model of [23] is restricted to a common Newtonian limit.

5. Entropy generation peaks near the walls; the Brinkman number and magnetic parameter raise friction and Joule irreversibility; hybrid loading (total φ up to 0.10) increases the Nusselt number by approximately 41.2%, accompanied by an approximately 31.7% increase in the average entropy-generation number N_s,avg (from 3.1844 to 4.1940).

# Appendix A. Reduced and Limiting Forms

In the Newtonian limit (n = 1 or We → 0) under the adopted simplified Carreau model with μ_∞ = 0, the Carreau bracket [1 + We² f″²]^{(n−1)/2} becomes unity and the momentum equation (23) reduces to the Wang squeezing-film form stated in Section 5. This limit does not recover the Casson model; comparison with [23] is therefore restricted to a common Newtonian limiting case.

# References

[1] S. U. S. Choi, J. A. Eastman, Enhancing thermal conductivity of fluids with nanoparticles, in: Developments and Applications of Non-Newtonian Flows, ASME International Mechanical Engineering Congress and Exposition, San Francisco, CA, USA, 12–17 Nov. 1995, ASME FED-Vol. 231/MD-Vol. 66, pp. 99–105.

[2] M. R. Eid, A. F. Al-Hossainy, Combined Newtonian heating and chemical reaction effects on hybrid nanofluid flow, Waves in Random and Complex Media 33 (2023) 1–26. https://doi.org/10.1080/17455030.2021.1897708

[3] J. Buongiorno, Convective transport in nanofluids, ASME Journal of Heat Transfer 128 (3) (2006) 240–250. https://doi.org/10.1115/1.2150834

[4] S. Suresh, K. P. Venkitaraj, P. Selvakumar, M. Chandrasekar, Synthesis of Al₂O₃–Cu/water hybrid nanofluids using two-step method and its thermo-physical properties, Colloids and Surfaces A: Physicochemical and Engineering Aspects 388 (2011) 41–48. https://doi.org/10.1016/j.colsurfa.2011.08.005

[5] D. K. Mandal, N. Biswas, N. K. Manna, R. S. R. Gorla, A. J. Chamkha, Hybrid nanofluid flow in a complex cavity, International Journal of Numerical Methods for Heat & Fluid Flow 33 (6) (2023) 2111–2143. https://doi.org/10.1108/HFF-07-2022-0444

[6] N. K. Manna, N. Biswas, D. K. Mandal, U. K. Sarkar, H. F. Öztop, N. Abu-Hamdeh, Magneto-thermal convection of hybrid nanofluid, International Journal of Numerical Methods for Heat & Fluid Flow 33 (4) (2023) 1249–1286. https://doi.org/10.1108/HFF-07-2022-0402

[7] F. Afshari, B. Muratçobanoğlu, Thermal analysis of a hybrid nanofluid, International Journal of Environmental Science and Technology 20 (2) (2023) 2037–2052. https://doi.org/10.1007/s13762-022-04144-5

[8] I. Tlili, M. T. Mustafa, K. A. Kumar, N. Sandeep, Effect of asymmetrical heat rise/fall on the AA7072–AA7075/methanol hybrid nanofluid flow, Scientific Reports 10 (2020) 4402. https://doi.org/10.1038/s41598-020-61215-8

[9] A. Mishra, K. Swain, S. Dash, Soret and Dufour effects on hybrid nanofluid flow, Journal of Computational Applied Mechanics 53 (1) (2022) 1–14. https://doi.org/10.22059/jcamech.2022.332096.656

[10] Zeeshan, N. Ahmad, F. Mabood, M. Alghamdi, Entropy analysis of EMHD nanofluid flow, Scientific Reports 13 (2023) 9981. https://doi.org/10.1038/s41598-023-37100-5

[11] G. Ashwinkumar, S. Sulochana, C. Sulochana, Effect of the aligned magnetic field on the boundary-layer analysis of the magnetic nanofluid over a stretching sheet, Alexandria Engineering Journal 58 (4) (2019) 1461–1470. https://doi.org/10.1016/j.aej.2019.11.013

[12] P. J. Carreau, Rheological equations from molecular network theories, Transactions of the Society of Rheology 16 (1) (1972) 99–127. https://doi.org/10.1122/1.549276

[13] S. A. G. A. Shah, A. Hassan, H. Karamti, A. Alhushaybari, S. M. Eldin, A. M. Galal, Effect of thermal radiation on Carreau nanofluid, Scientific Reports 13 (2023) 4117. https://doi.org/10.1038/s41598-023-31152-3

[14] H. A. Wahab, S. Zeb, S. Khan, M. Zamir, S. O. Alharbi, Numerical study of Carreau nanofluid flow, Arabian Journal of Chemistry 16 (5) (2023) 104682. https://doi.org/10.1016/j.arabjc.2023.104682

[15] M. P. Mkhatshwa, M. Khumalo, Irreversibility analysis of Carreau fluid flow, Heat Transfer 52 (1) (2023) 395–429. https://doi.org/10.1002/htj.22700

[16] A. S. Mittal, H. R. Patel, Influence of thermophoresis and Brownian motion on Carreau fluid, Physica A: Statistical Mechanics and its Applications 537 (2020) 122710. https://doi.org/10.1016/j.physa.2019.122710

[17] M. Qayyum, S. Afzal, E. Ahmad, M. B. Riaz, Fractional modelling of Carreau fluid, Coatings 12 (11) (2022) 1680. https://doi.org/10.3390/coatings12111680

[18] M. J. Stefan, Versuche über die scheinbare Adhäsion, Sitzungsberichte der Kaiserlichen Akademie der Wissenschaften Wien, Mathematisch-Naturwissenschaftliche Classe 69 (1874) 713–721.

[19] R. J. Grimm, Squeezing flows of Newtonian liquid films, Applied Scientific Research 32 (2) (1976) 149–166. https://doi.org/10.1007/BF00385167

[20] G. M. Sobamowo, A. T. Akinshilo, On the analysis of squeezing flow of nanofluid between two parallel plates, Alexandria Engineering Journal 57 (3) (2018) 1413–1423. https://doi.org/10.1016/j.aej.2017.06.022

[21] S. Ahmad, M. Farooq, M. Javed, A. Anjum, Slip analysis of squeezing flow, Results in Physics 9 (2018) 527–533. https://doi.org/10.1016/j.rinp.2018.02.066

[22] S. Ahmad, M. Farooq, M. Javed, A. Anjum, Double stratification effects in chemically reactive squeezed flow, Results in Physics 8 (2018) 1250–1259. https://doi.org/10.1016/j.rinp.2018.01.043

[23] K. Bhaskar, K. Sharma, Entropy generation of Casson fluid flow, Indian Journal of Physics 95 (7) (2021) 1453–1467. https://doi.org/10.1007/s12648-020-01800-9

[24] C. Soret, Sur l'état d'équilibre que prend une dissolution saline, Archives des Sciences Physiques et Naturelles 2 (1879) 48–61.

[25] E. R. G. Eckert, R. M. Drake, Analysis of Heat and Mass Transfer, McGraw-Hill, New York, 1972.

[26] A. Shojaei, A. J. Amiri, S. S. Ardahaie, K. Hosseinzadeh, D. D. Ganji, Soret and Dufour effects on hybrid nanofluid flow, Case Studies in Thermal Engineering 13 (2019) 100384. https://doi.org/10.1016/j.csite.2018.100384

[27] K. Rafique, M. I. Anwar, M. Misiran, I. Khan, S. O. Alharbi, P. Thounthong, K. S. Nisar, Keller-box analysis of Soret and Dufour effects, Frontiers in Physics 7 (2019) 139. https://doi.org/10.3389/fphy.2019.00139

[28] R. N. Kumar, R. J. P. Gowda, B. C. Prasannakumara, Soret and Dufour effects on radiative nanofluid flow, Indian Journal of Physics 97 (3) (2023) 775–785. https://doi.org/10.1007/s12648-022-02428-7

[29] B. K. Sharma, R. Gandhi, N. K. Mishra, Q. M. Al-Mdallal, Entropy generation in MHD nanofluid flow with Joule heating, Nanomaterials 13 (3) (2023) 544. https://doi.org/10.3390/nano13030544

[30] M. M. Bhatti, O. A. Bég, R. Ellahi, T. Abbas, Natural convection non-Newtonian EMHD flow, Qualitative Theory of Dynamical Systems 21 (3) (2022) 97. https://doi.org/10.1007/s12346-022-00625-7

[31] R. Gandhi, B. K. Sharma, N. K. Mishra, Q. M. Al-Mdallal, Computer simulations of EMHD Casson nanofluid flow, Nanomaterials 13 (4) (2023) 652. https://doi.org/10.3390/nano13040652

[32] B. Mahanthesh, J. Mackolil, N. S. Shashikumar, Nonlinear radiative heat transfer with Darcy–Forchheimer flow, Journal of Thermal Analysis and Calorimetry 141 (2020) 37–44. https://doi.org/10.1007/s10973-019-09097-5

[33] A. Shahzad, M. Imran, M. Tahir, F. Khan, A. Akgül, S. M. Eldin, Brownian motion and thermophoresis effects in Darcy–Forchheimer flow, Alexandria Engineering Journal 62 (2023) 1–15. https://doi.org/10.1016/j.aej.2022.07.023

[34] A. Bejan, A study of entropy generation in fundamental convective heat transfer, ASME Journal of Heat Transfer 101 (4) (1979) 718–725. https://doi.org/10.1115/1.3451063

[35] A. Bejan, Entropy Generation Minimization: The Method of Thermodynamic Optimization of Finite-Size Systems and Finite-Time Processes, CRC Press, Boca Raton, FL, 1996.

[36] M. I. Khan, S. Qayyum, T. Hayat, M. I. Khan, A. Alsaedi, T. A. Khan, Entropy generation in radiative motion of tangent hyperbolic nanofluid, International Journal of Heat and Mass Transfer 133 (2019) 959–967. https://doi.org/10.1016/j.ijheatmasstransfer.2018.12.168

[37] S. Rashidi, J. A. Esfahani, M. Maskaniyan, Applications of magnetohydrodynamics in biological systems, Journal of Magnetism and Magnetic Materials 439 (2017) 358–372. https://doi.org/10.1016/j.jmmm.2017.05.014

[38] M. M. Bhatti, A. Zeeshan, R. Ellahi, Endoscope analysis on peristaltic blood flow, Journal of Magnetics 21 (3) (2016) 468–475. https://doi.org/10.4283/JMAG.2016.21.3.468

[39] T. Siva, S. Jangili, B. Kumbhakar, Entropy generation in EMHD nanofluid flow through a microchannel, International Journal of Thermal Sciences 191 (2023) 108339. https://doi.org/10.1016/j.ijthermalsci.2023.108339

[40] S. Bhatti, M. Zeeshan, R. Ellahi, O. A. Bég, A. Kadir, Entropy generation in peristaltic nanofluid flow, Journal of Thermal Analysis and Calorimetry 147 (2022) 1–17. https://doi.org/10.1007/s10973-021-10681-x

[41] A. Ali, H. Sarwar, S. Bukhari, Z. Abbas, Impact of thermal radiation on entropy generation, Waves in Random and Complex Media 33 (2023) 1–29. https://doi.org/10.1080/17455030.2021.1927237

[42] P. K. Yadav, A. Kumar, Entropy generation of nanofluid flow in a porous channel, International Communications in Heat and Mass Transfer 128 (2021) 105632. https://doi.org/10.1016/j.icheatmasstransfer.2021.105632

[43] N. K. Mishra, Entropy generation analysis of MHD nanofluid flow, Acta Mechanica et Automatica 17 (1) (2023) 1–8. https://doi.org/10.2478/ama-2023-0001