Challenges in Machining Biomaterials

| Metallic Biomaterials | Ceramic Biomaterials | Polymeric Biomaterials | Composite Biomaterials |
| --- | --- | --- | --- |
| • Low thermal conductivity • High strength and work hardening • High cutting forces • Tool wear: diffusion and adhesion • Built-up edge formation • Poor surface finish • Heat concentration in cutting zone | • High hardness • Low fracture toughness • Brittle nature • Prone to cracking and chipping • Surface and subsurface damage • Low machining efficiency • Requires non-traditional machining methods | • Low melting point • Low thermal conductivity • Heat accumulation in cutting zone • Thermal softening and melting • Burr formation • Viscoelastic behavior • Dimensional inaccuracy • Poor surface finish | • Heterogeneous structure • Non-uniform cutting forces • Fiber pull-out • Matrix cracking • Delamination • Surface damage • Difficult to predict machining behavior • Requires optimized cutting parameters |