Influence of Biomaterial Properties on Machining Performance

Higher values indicate greater effect/challenge on machinability metrics (Scale: Low / Medium / High)

■ Low Effect / Challenge     ■ Medium Effect / Challenge     ■ High Effect / Challenge

| (A) Effect on Cutting Force Property; 1; 2; 3; Level. Hardness; High (3). Strength; High (3). Toughness (Ductility); Medium (2). Thermal Conductivity; High (3). Grain Size; Medium (2). Phase Composition; Medium (2). Friction & Adhesion; Medium (2). Density; Low (1). Note: Higher hardness, strength and lower thermal conductivity increase cutting forces significantly [1,2,6,31,32,36] | (B) Effect on Tool Wear Property; 1; 2; 3; Level. Hardness; High (3). Thermal Conductivity; High (3). Friction & Adhesion; High (3). Abrasion (2nd Phase Particles); High (3). Chemical Reactivity; Medium (2). Phase Composition; Medium (2). Toughness (Ductility); Low (1). Density; Low (1). Note: High hardness, low thermal conductivity, adhesion, abrasives and reactivity lead to severe tool wear [1,6,8,9,31,41,43,44] |
| --- | --- |
| (C) Effect on Surface Roughness / Integrity Property; 1; 2; 3; Level. Grain Size; High (3). Phase Composition; Medium (2). Friction & Adhesion; Medium (2). Thermal Conductivity; Medium (2). Toughness (Ductility); Low (1). Tool Material & Geometry; Low (1). Cutting Parameters; Low (1). Cooling / Lubrication; Low (1). Note: Finer grains, phase changes and high adhesion worsen surface quality; proper tools and cooling improve it [11,25,26,27,33,39] | (D) Effect on Chip Formation Property; 1; 2; 3; Level. Toughness (Ductility); High (3). Hardness; Medium (2). Thermal Conductivity; Medium (2). Friction & Adhesion; Medium (2). Phase Composition; Medium (2). Cutting Parameters; Low (1). Cooling / Lubrication; Low (1). Tool Geometry; Low (1). Note: Ductile materials form continuous chips; brittle materials produce fractured/powdery chips [3,4,10,33,34] |

Key Takeaway:  Mechanical (hardness, strength), thermal (low thermal conductivity), microstructural (grain size, phases, abrasives) and tribological (friction, adhesion, reactivity) properties are the dominant factors affecting machinability of biomaterials.