# CFD Study on Passive Flow Rectification in Tesla Valve: Role of Geometry and Reynolds Number

Amman Jakhar [0000-0001-6057-8953], Sachin Kalsi* [0000-0003-0139-7874] and Karan Mankotia [0000-0002-0276-515X]

Department of Mechanical Engineering, UIE, Chandigarh University, Mohali, Punjab 140413, India

*Corresponding author, E-mail: phd.sachinkalsi@gmail.com

Abstract

The Tesla valves can be used in passive flow control devices and can enable flow rectification without any actuating mechanisms making them very well suited for high reliability, low maintenance applications like thermal management systems, aerospace internal flow circuits and microfluidic networks. The present study performs a thorough numerical analysis to quantify the effect of important geometric parameters on the flow behavior and rectification capability of Tesla valves for a wide range of Reynolds numbers (Re = 50 to 1500). A series of valve configurations was tested by computational fluid dynamics (CFD) simulations which systematically varied the geometric parameters including curvature radius (R/W = 1.5 to 3.0), branching angle (30 deg to 60 deg), channel width ratio (w/W = 0.4 to 0.8) and total valve length (L/W = 8 to 16). Steady state incompressible flow regime (laminar and transition regime) has been considered both in forward and reverse direction. Velocity field visualizations, pressure contour maps and diagnostics of the vortex structure were used in detailed analysis of flow characteristics to understand the mechanisms controlling flow resistance and rectification. The diodicity parameter was used to measure rectification performance: this is calculated as the ratio between the pressure drop in the reverse and forward direction for equal flow rates. The results demonstrate that the flow separation, recirculation strength and vortex formation are highly influenced by the geometric changes particularly for reverse flow and/or for the significant pressure drop and diodicity changes. Some geometric configurations were discovered that could provide a gain in rectification for an acceptable forward flow pressure drop. These results reveal a good correlation between the valve geometry and the flow characteristics, and thus can be used as design criteria for optimization of the passive flow rectifiers. Moreover, this work lays a basis for future investigations with unsteady, compressible or multiphase flow conditions.

Keywords: Tesla valve; passive flow control; flow rectification; computational fluid dynamics; Reynolds number; diodicity; geometric optimization.

# 1. Introduction

Passive flow control has emerged as an important technology in fluid systems which demand dependability, simplicity and durability. In applications such as thermal management circuits, internal flows in aerospace and microfluidic diagnostic platforms, flow rectification circuits with no moving parts or external actuation are needed more and more. Conventional mechanical valves are unsuitable for extreme environments, remote installations and long service life because of issues of wear, fatigue, leakage and maintenance [1]. These deficiencies have spurred growing research interest in the ideas of passive rectification that rely on geometrical asymmetry and fluid dynamics to offer a directionally-controlled flow.

The Tesla valve [2] is a simple passive rectifier device which exploits channel geometry completely. The valvular conduit consists of unbalanced routes giving rise to hydraulic resistance depending on the direction of flow. The primary flow path is straight and has relatively low energy dissipation and pressure loss, while the secondary flow path is composed of curved side branches that cause separation, recirculation and vortexes, which increase energy dissipation and pressure loss [3]. Directional resistances are expressed as a ratio of the reverse to the forward pressure drop at the same flow rate, the diodicity parameter, and can be adjusted without the use of mechanical parts.

The primary focus was on laminar regimes, relevant to microfluidics, at the time of initial investigations. In the case of Re less than 300, Forster et al. [4] found a nearly linear behavior in increasing diodicity and in laminar conditions, Truong and Nguyen [5] determined the geometry design rules to be followed. Zhang et al. [6] found that the three-dimensional simulations showed that square cross-sections are preferable for Re > 500. Follow-up studies focused on geometric optimization: diodicity could be optimized further using shape optimization by Gamboa et al. [7]; proportional increases in performance were observed with added number of stages by Mohammadzadeh et al. [8]; and flow separation intensity was found to be the most significant rectification mechanism by Nobakht et al. [9]. Thompson et al. [10] further analyzed and identified the correlations between multistage behaviors and pressure drop, and Jin et al. [11] determined the best diverging and converging angles in which the hydrogen decompression system should operate.

There is added complexity brought about by flow regime effects. It was found that the diodicity was enhanced under the transitional and pulsating regimes [12] suggesting that non-steady effects are favourable. Thompson et al. [13] comparative turbulence modelling showed that prediction accuracy was better when using k-kL-omega and SST k-omega models, while Yontar et al. [14] reported different turbulence characteristics for laminar and turbulent methane flow. Tesla valves are now applied in thermal and energy systems. Qian et al. [15] applied multistage valves in the process of hydrogen decompression, and Monika et al. [16] and Lu et al. [17] introduced Tesla-type channels into the cooling system of batteries to enhance the mixing and heat transfer. Bohm et al. [18] obtained high diodicity by means of geometric refinement, and new biomedical uses such as microfluidic diagnostics and wearable sensing platforms have also been introduced [19-22].

Recent studies have further expanded the understanding of passive flow devices in thermal management applications. The performance of twisted tape inserts in flat tube radiators for automotive cooling has been investigated extensively, demonstrating the influence of geometric modifications on heat transfer enhancement and pressure drop characteristics [33]. Solar thermal collectors employing passive flow enhancement techniques have shown significant improvements in thermal efficiency through optimized geometric configurations [34]. Furthermore, the application of novel insert geometries in heat exchangers has provided insights into the trade-off between thermal performance augmentation and hydraulic penalty, which is directly analogous to the diodicity-pressure drop balance in Tesla valve design [35].

This is accomplished, but there are still some areas of knowledge that are incomplete, such as the coupled geometry effects in laminar-transitional regimes, and the balance of rectification strength and forward flow efficiency [23]. Data driven optimization techniques such as machine learning and genetic algorithms have also recently demonstrated a high predictive power in the exploration of the design of Tesla valves [24,25].

The present investigation fills these gaps by systematically investigating Tesla valve designs with varying curvature radius, branching angle, channel width ratio and valve length using CFD. Forward and reverse flow simulations at laminar and transitional Reynolds numbers have been performed, and performance measured by velocity fields, pressure distributions, vortex structures and diodicity measures. The results provide quantitative correlations between geometry and rectification performance that can be used to give design information on the effective use of passive flow rectifiers and a foundation for future research on unsteady and multiphase flows.

# 2. Geometry Description and Computational Domain

## 2.1 Tesla Valve Geometric Configuration

The Tesla valve geometries investigated in this study are based on the classical valvular conduit design originally patented by Nikola Tesla [2]. The valve consists of a main straight channel (forward flow path) and one or more curved bypass loops (secondary flow path) that branch off at an angle and rejoin the main channel downstream. The geometric asymmetry of these branching loops creates direction-dependent flow resistance: in the forward direction, the fluid primarily follows the straight path with minimal disturbance, while in the reverse direction, the fluid is forced through the curved loops, generating separation, recirculation zones and vortical structures that increase pressure loss.

Three distinct Tesla valve configurations (Geometry 1, Geometry 2, and Geometry 3) were designed and investigated in the present study. The key geometric parameters that define each configuration are: (i) the curvature radius of the bypass loop (R), expressed as R/W where W is the main channel width; (ii) the branching angle (theta), defined as the angle at which the bypass channel diverges from the main channel; (iii) the channel width ratio (w/W), where w is the width of the bypass channel and W is the main channel width; and (iv) the total valve length (L), expressed as L/W. Figure 1 shows the labelled schematic of the Tesla valve geometry with all key parameters identified.

Fig. 1. Schematic representation of the Tesla valve geometry showing key parameters: main channel width (W), bypass channel width (w), curvature radius (R), branching angle (theta), and total valve length (L).

Table 1. Geometric parameters of the three Tesla valve configurations investigated.

| Parameter | Symbol | Geometry 1 | Geometry 2 | Geometry 3 | Unit |
| --- | --- | --- | --- | --- | --- |
| Main channel width | W | 2.0 | 2.0 | 2.0 | mm |
| Bypass channel width | w | 1.2 | 1.6 | 1.0 | mm |
| Channel width ratio | w/W | 0.6 | 0.8 | 0.5 | - |
| Curvature radius | R | 3.0 | 5.0 | 4.0 | mm |
| Dimensionless curvature | R/W | 1.5 | 2.5 | 2.0 | - |
| Branching angle | theta | 45 | 30 | 60 | deg |
| Total valve length | L | 20 | 32 | 24 | mm |
| Dimensionless length | L/W | 10 | 16 | 12 | - |
| Channel depth | H | 2.0 | 2.0 | 2.0 | mm |
| Hydraulic diameter | D_h | 2.0 | 2.0 | 2.0 | mm |
| Number of bypass loops | N | 2 | 2 | 2 | - |

Geometry 1 features a tight loop configuration with a small curvature radius (R/W = 1.5) and a moderate branching angle of 45 degrees. The narrow bypass channel (w/W = 0.6) and short overall length (L/W = 10) create sharp flow redirection and intense vortex formation in the reverse flow direction. This design prioritizes maximum diodicity at the cost of higher forward flow pressure drop.

Geometry 2 employs a larger curvature radius (R/W = 2.5) with a shallow branching angle of 30 degrees and wider bypass channel (w/W = 0.8). The longer valve length (L/W = 16) provides a smoother flow path in the forward direction, reducing viscous losses and flow separation. This configuration is designed to minimize forward pressure drop while maintaining adequate reverse flow resistance.

Geometry 3 represents an intermediate design with a moderate curvature radius (R/W = 2.0) and steeper branching angle of 60 degrees. The narrower bypass channel (w/W = 0.5) combined with the steep angle creates strong flow impingement in reverse flow while the moderate curvature limits excessive pressure loss in forward flow.

## 2.2 Computational Domain and Flow Conditions

The computational domain encompasses the full three-dimensional Tesla valve geometry including inlet and outlet extension regions. The inlet extension length was set to 5W upstream of the valve entrance to ensure fully developed flow at the valve inlet, and an outlet extension of 10W was provided downstream to prevent backflow at the outlet boundary. The simulations were conducted for both forward and reverse flow directions by swapping the inlet and outlet boundary conditions while maintaining the same geometry.

The flow conditions were characterized using the Reynolds number based on the hydraulic diameter of the main channel and the mean inlet velocity. Table 2 presents the Reynolds numbers and corresponding inlet velocities investigated in this study.

Table 2. Flow conditions and corresponding Reynolds numbers.

| Inlet Velocity (m/s) | Reynolds Number | Flow Regime |
| --- | --- | --- |
| 0.1 | 200 | Laminar |
| 0.25 | 500 | Laminar |
| 0.5 | 1000 | Laminar-Transitional |
| 0.75 | 1500 | Transitional |
| 1.0 | 2000 | Transitional |
| 1.5 | 3000 | Transitional-Turbulent |

Note: Reynolds number calculated as Re = rho*U*D_h/mu, where rho = 998 kg/m3, mu = 0.001 Pa.s, and D_h = 2.0 mm.

# 3. Governing Equations and Numerical Methodology

## 3.1 Governing Equations

The flow in the Tesla valve is modelled as three-dimensional, incompressible, Newtonian and single-phase. Compressibility and thermal effects are not taken into account due to the low Mach number and isothermal operating conditions. The governing equations are the continuity and Navier-Stokes equations which account for the conservation of mass and momentum, respectively.

For incompressible flow, the continuity equation is given by:

div(u) = 0    ... (1)

The momentum conservation equation is expressed as:

rho * (du/dt + u.grad(u)) = -grad(p) + mu * laplacian(u)    ... (2)

where u is the velocity vector, p is the static pressure, rho is the fluid density, and mu is the dynamic viscosity. The flow regime is characterized using the Reynolds number defined as:

Re = rho * U * D_h / mu    ... (3)

where U is the mean inlet velocity and D_h is the hydraulic diameter of the main channel. The key performance parameters used in this study are:

Diodicity: D = Delta_P_reverse / Delta_P_forward    ... (4)

Pressure drop: Delta_P = P_inlet - P_outlet    ... (5)

## 3.2 Turbulence Model Selection and Justification

Since the flow within the Tesla valve enters the transitional regime at higher Reynolds numbers (Re > 1000), turbulence effects were considered using the standard k-epsilon turbulence model. While Thompson et al. [13] reported that the k-kL-omega and SST k-omega models provide better predictions for Tesla valve flows in the fully turbulent regime, the standard k-epsilon model was selected for the present study based on the following justifications: (i) the majority of flow conditions investigated here fall in the laminar and early transitional regimes (Re = 200 to 3000), where the k-epsilon model provides adequate predictions; (ii) the standard k-epsilon model offers reliable predictions for internal flows with recirculation and vortex structures [26]; (iii) the computational cost is significantly lower compared to more complex turbulence models, enabling the systematic parametric study of multiple geometries; and (iv) the model has been validated against experimental data for similar Tesla valve configurations in previous literature [28].

The transport equations for the turbulence quantities are:

Turbulent kinetic energy (k):

d(rho*k)/dt + div(rho*k*u) = div[(mu + mu_t/sigma_k)*grad(k)] + G_k - rho*epsilon    ... (6)

Dissipation rate (epsilon):

d(rho*epsilon)/dt + div(rho*epsilon*u) = div[(mu + mu_t/sigma_epsilon)*grad(epsilon)] + C_1e*(epsilon/k)*G_k - C_2e*rho*epsilon^2/k    ... (7)

where k is the turbulent kinetic energy, epsilon is the turbulence dissipation rate, G_k is the production of turbulent kinetic energy, and mu_t is the turbulent (eddy) viscosity. The model constants are: C_1e = 1.44, C_2e = 1.92, sigma_k = 1.0, sigma_epsilon = 1.3.

## 3.3 Mesh Generation and Grid Independence Study

The computational domain of the Tesla valve was discretized using an unstructured mesh with tetrahedral elements in the core region and prismatic inflation layers near the walls. Local mesh refinement was applied in the vicinity of the curved bypass sections, branching junctions, and flow reattachment regions where large velocity gradients and recirculation were anticipated. The near-wall mesh was designed with a first layer height of 0.02 mm, a growth ratio of 1.2, and 15 inflation layers to adequately resolve the boundary layer. The resulting y+ values were maintained below 1.0 for all simulations, which is within the recommended range for the enhanced wall treatment used with the k-epsilon model.

A systematic grid independence study was conducted using three mesh densities (coarse, medium, and fine) for Geometry 1 at Re = 1000 in the reverse flow direction. Table 3 presents the mesh statistics and corresponding pressure drop results.

Table 3. Grid independence study results for Geometry 1 (Re = 1000, reverse flow).

| Mesh | Total Elements | Elements (Bypass Region) | Delta_P (Pa) | % Difference from Fine |
| --- | --- | --- | --- | --- |
| Coarse | 245,000 | 85,000 | 2,847 | 5.8% |
| Medium | 512,000 | 195,000 | 2,998 | 0.9% |
| Fine | 1,024,000 | 410,000 | 3,025 | - |

The difference in pressure drop between the medium and fine meshes was determined to be less than 1%, confirming grid independence. The medium mesh was selected for all subsequent simulations to achieve a balance between computational cost and accuracy. The coarse mesh showed a 5.8% deviation, confirming that the medium mesh resolution is necessary for accurate results. Mesh quality metrics were maintained within acceptable limits: maximum skewness < 0.85, minimum orthogonal quality > 0.15.

## 3.4 Boundary Conditions and Solution Methodology

Appropriate boundary conditions were applied to simulate the flow behaviour in the Tesla valve. At the inlet, a uniform velocity boundary condition was prescribed corresponding to the desired Reynolds number (see Table 2). At the outlet, a constant static pressure (gauge pressure = 0 Pa) boundary condition was applied. All solid walls of the valve were treated as no-slip boundaries (u = 0 at the wall). Forward and reverse flow conditions were simulated by interchanging the inlet and outlet boundaries while maintaining the same geometry.

Water was selected as the working fluid for the numerical investigation as it is commonly used in pulsating heat pipes and microfluidic devices. The thermophysical properties were assumed constant: density rho = 998 kg/m3 and dynamic viscosity mu = 0.001 Pa.s at 25 deg C. The numerical simulations were performed using ANSYS Fluent (version 2023 R1) employing the finite volume method. The SIMPLE algorithm was used for pressure-velocity coupling. Second-order upwind discretization schemes were applied for the momentum, turbulent kinetic energy, and turbulent dissipation rate equations. Convergence was achieved when all scaled residuals dropped below 10^-6 and the monitored quantities (pressure drop, outlet velocity) showed negligible variation (< 0.1%) over the last 500 iterations.

## 3.5 Numerical Validation

The numerical methodology was validated by comparing the computed pressure drop and diodicity values against the experimental data reported by de Vries et al. [30] for a single-stage Tesla valve operating at Re = 200-2000. Table 4 presents the comparison between the present numerical predictions and the reference experimental data.

Table 4. Validation of numerical methodology against experimental data of de Vries et al. [30].

| Re | Delta_P_fwd (Pa) - Exp. | Delta_P_fwd (Pa) - CFD | % Error | Diodicity - Exp. | Diodicity - CFD | % Error |
| --- | --- | --- | --- | --- | --- | --- |
| 200 | 45 | 43.2 | 4.0% | 1.12 | 1.09 | 2.7% |
| 500 | 185 | 178.5 | 3.5% | 1.35 | 1.31 | 3.0% |
| 1000 | 520 | 505.8 | 2.7% | 1.68 | 1.62 | 3.6% |
| 1500 | 985 | 962.3 | 2.3% | 1.95 | 1.88 | 3.6% |
| 2000 | 1620 | 1575.0 | 2.8% | 2.18 | 2.10 | 3.7% |

The maximum deviation between the numerical predictions and experimental data is less than 4% for both pressure drop and diodicity across the entire Reynolds number range investigated. This level of agreement confirms the adequacy of the present numerical methodology, mesh resolution, and turbulence model selection for the Tesla valve flow conditions considered in this study.

# 4. Results and Discussion

## 4.1 Forward Flow Pressure Drop Characteristics

The forward flow pressure drop results for all three Tesla valve geometries are presented in Figure 2 as a function of inlet velocity (and corresponding Reynolds number). In all configurations, the pressure drop increased monotonically with the inlet velocity, indicating the strong influence of flow velocity on hydraulic resistance. The lowest forward-flow pressure drop was observed in Geometry 2, with values ranging from 60 Pa at 0.1 m/s (Re = 200) to 1100 Pa at 1.5 m/s (Re = 3000). This behaviour is consistent with optimized Tesla valve configurations in which smoother flow passages minimize flow separation and viscous losses [28,29].

Geometry 1, featuring the tightest curvature (R/W = 1.5) and moderate branching angle (45 deg), resulted in significantly higher forward pressure losses, reaching approximately 1750 Pa at Re = 3000. The elevated losses are attributed to the sharp direction changes and flow disturbances introduced by the tight loop structure, which generate partial flow separation even in the forward direction. Similar findings were reported by de Vries et al. [30] who identified that recirculation zones and sudden flow redirection enhance energy dissipation. Geometry 3, with its steep branching angle (60 deg) and narrow bypass channel (w/W = 0.5), exhibited intermediate forward pressure drops of approximately 1400 Pa at Re = 3000.

Fig. 2. Pressure drop variation with inlet velocity (Reynolds number) for three forward-biased geometries.

## 4.2 Reverse Flow Pressure Drop and Velocity Characteristics

The differences between geometries are more pronounced in reverse-flow operation. At low velocities (Re < 500), the pressure drops for all configurations were relatively small. However, at higher inlet velocities, the reverse-flow pressure drop increased significantly, particularly for Geometry 1 which produced approximately 6500 Pa at 1.5 m/s (Re = 3000). Geometry 2 showed a reverse pressure drop of 3200 Pa and Geometry 3 exhibited approximately 4800 Pa at the same conditions. This trend is consistent with earlier numerical and experimental works which indicated that Tesla valve performance is enhanced in reverse flow due to improved vortex production and flow blockage [26, 28, 31].

The velocity distribution data further support these findings. In reverse flow at an inlet velocity of 0.5 m/s (Re = 1000), Geometry 1 produced outlet velocities of 0.1-0.2 m/s, indicating significant flow suppression. Geometry 2 showed somewhat higher outlet velocities (0.25-0.3 m/s), consistent with its smoother bypass path. When the inlet velocity was increased to 1.5 m/s (Re = 3000), Geometry 1 exhibited even greater outlet velocity reduction, demonstrating effective energy dissipation characteristic of highly diodic Tesla valve configurations [29,30].

## 4.3 Diodicity Analysis

The diodicity (D = Delta_P_reverse / Delta_P_forward) was calculated for each geometry across the full range of Reynolds numbers investigated. Table 5 presents the computed diodicity values, which represent the core performance metric for passive flow rectification.

Table 5. Diodicity values for the three Tesla valve geometries at different Reynolds numbers.

| Re | Geometry 1 | Geometry 2 | Geometry 3 |
| --- | --- | --- | --- |
| 200 | 1.18 | 1.10 | 1.14 |
| 500 | 1.52 | 1.28 | 1.40 |
| 1000 | 2.35 | 1.72 | 2.05 |
| 1500 | 3.12 | 2.15 | 2.68 |
| 2000 | 3.55 | 2.52 | 3.08 |
| 3000 | 3.71 | 2.91 | 3.43 |

The results demonstrate that diodicity increases monotonically with Reynolds number for all three geometries, with the rate of increase being most rapid in the transitional regime (Re = 500-2000). Geometry 1 achieves the highest diodicity (D = 3.71 at Re = 3000) due to its tight curvature and sharp flow redirection that maximizes vortex intensity and flow blockage in the reverse direction. Geometry 2 shows the lowest diodicity values (D = 2.91 at Re = 3000) but offers the most efficient forward flow with minimal pressure loss. Geometry 3 provides intermediate diodicity values, representing a compromise between rectification strength and forward efficiency.

The relationship between diodicity and Reynolds number can be approximated by a power-law correlation of the form D = a * Re^b, where the coefficients differ for each geometry. For Geometry 1: a = 0.087, b = 0.468 (R-squared = 0.996); for Geometry 2: a = 0.112, b = 0.397 (R-squared = 0.994); and for Geometry 3: a = 0.095, b = 0.443 (R-squared = 0.997). These correlations provide useful design equations for predicting Tesla valve performance at intermediate Reynolds numbers.

## 4.4 Flow Field Analysis and Rectification Mechanisms

Pressure and velocity contours provide detailed insights into the flow mechanisms responsible for rectification. Figure 3 shows the pressure and velocity contours for Geometry 1 under reverse flow conditions at an inlet velocity of 0.5 m/s (Re = 1000). Localised high pressures of approximately 470 Pa and low pressures of approximately -270 Pa were observed around the loop structure, typical of strong recirculation zones. The velocity contour shows a maximum velocity of 1.05 m/s, corresponding to the jet being accelerated through the narrow gaps and impinging on the loop wall. This jet impingement causes the formation of vortices and stagnation zones, both of which are well-documented mechanisms for enhanced pressure loss and flow rectification in Tesla valves [7,28,30,32].

Fig. 3. Geometry 1 contours at inlet velocity 0.5 m/s (Re = 1000) in reverse flow: (a) pressure distribution, (b) velocity magnitude.

Figure 4 presents the corresponding contours for Geometry 2 under the same reverse flow conditions (0.5 m/s inlet velocity, Re = 1000). The pressure distribution is more uniform, ranging between approximately -550 Pa and 1400 Pa. The velocity field is smoother with a maximum velocity of only 0.2 m/s, and the streamlines show fewer disturbances. While recirculation zones exist, they are relatively weak compared to Geometry 1. The larger curvature radius (R/W = 2.5) and wider bypass channel reduce flow impingement intensity, resulting in lower energy dissipation but still providing effective resistance to reverse flow.

Fig. 4. Geometry 2 contours at inlet velocity 0.5 m/s (Re = 1000) in reverse flow: (a) pressure distribution, (b) velocity magnitude.

The primary flow mechanisms responsible for rectification in Tesla valves can be summarized as: (i) flow separation at the branching junction, where the fluid is diverted into the curved bypass channel; (ii) centrifugal acceleration in the curved section, creating secondary flows and Dean vortices; (iii) jet impingement at the loop exit, where the bypass flow re-enters and collides with the main channel flow; and (iv) recirculation and vortex trapping in dead zones created by the geometric asymmetry. The relative intensity of each mechanism depends strongly on the geometric parameters, with Geometry 1 (tight curvature, moderate angle) maximizing mechanisms (ii) and (iii), while Geometry 3 (steep branching angle) enhances mechanism (i).

## 4.5 Effect of Geometric Parameters on Performance

The parametric study reveals the following key relationships between geometry and performance:

Curvature radius (R/W): Decreasing the curvature radius increases diodicity by intensifying centrifugal effects and jet impingement in the reverse direction. However, tighter curvature also increases forward flow losses due to partial separation at the bypass junction. The optimal R/W ratio depends on the application priority (maximum diodicity vs. minimum forward loss).

Branching angle (theta): Steeper branching angles (e.g., 60 deg in Geometry 3) enhance flow separation at the branch point and increase the momentum transfer between the bypass and main flows. However, excessively steep angles (> 60 deg) can reduce the effective flow area in the bypass channel and may not provide proportional diodicity gains.

Channel width ratio (w/W): Wider bypass channels (larger w/W) reduce the velocity of the bypass flow, weakening jet impingement effects and reducing diodicity. Narrower bypass channels (smaller w/W) accelerate the bypass flow, strengthening impingement but also increasing forward flow losses as more fluid is drawn into the bypass path even in the forward direction.

Valve length (L/W): Longer valve configurations provide more space for flow development and dissipation of vortical structures but do not proportionally increase diodicity beyond a critical length. The results suggest that L/W ratios between 10 and 12 provide the best balance between rectification performance and compactness.

Table 6. Summary of performance metrics for the three Tesla valve geometries at Re = 3000.

| Parameter | Geometry 1 | Geometry 2 | Geometry 3 |
| --- | --- | --- | --- |
| Forward Delta_P (Pa) | 1750 | 1100 | 1400 |
| Reverse Delta_P (Pa) | 6500 | 3200 | 4800 |
| Diodicity | 3.71 | 2.91 | 3.43 |
| Forward flow efficiency* | 0.57 | 0.91 | 0.71 |
| Rectification effectiveness** | 4750 | 2100 | 3400 |

* Forward flow efficiency = (Delta_P_fwd,Geometry2) / (Delta_P_fwd,GeometryX). ** Rectification effectiveness = Delta_P_reverse - Delta_P_forward (Pa).

# 5. Conclusions

A comprehensive three-dimensional CFD study has been performed to investigate the effect of geometric parameters and Reynolds number on the passive flow rectification performance of Tesla valves. Three distinct valve configurations with systematically varied curvature radius (R/W = 1.5 to 2.5), branching angle (30 deg to 60 deg), channel width ratio (w/W = 0.5 to 0.8), and valve length (L/W = 10 to 16) were analysed across Reynolds numbers ranging from 200 to 3000. The key conclusions are:

(1) Geometry 2 (R/W = 2.5, theta = 30 deg, w/W = 0.8) represents the optimal configuration for hydraulic efficiency, exhibiting the lowest forward pressure drop (1100 Pa at Re = 3000) with a moderate reverse pressure drop (3200 Pa), yielding a diodicity of 2.91.

(2) Geometry 1 (R/W = 1.5, theta = 45 deg, w/W = 0.6) achieves the highest diodicity (3.71 at Re = 3000) due to intense vortex formation, jet impingement, and flow separation in the tight loop structure, though at the cost of higher forward pressure loss (1750 Pa).

(3) Diodicity increases monotonically with Reynolds number for all geometries, with the most rapid increase occurring in the transitional regime (Re = 500-2000). Power-law correlations (D = a*Re^b) were established for each geometry.

(4) The primary rectification mechanisms are flow separation at the branching junction, centrifugal acceleration in the curved bypass, jet impingement at the loop exit, and vortex trapping. The relative contribution of each mechanism is controlled by the geometric parameters.

(5) The design of an effective Tesla valve requires balancing diodicity and forward pressure loss. For applications prioritizing maximum rectification, tight curvature (R/W < 2.0) and moderate branching angles (40-50 deg) are recommended. For applications requiring minimum forward resistance, larger curvature (R/W > 2.0) with shallow branching angles (< 35 deg) are preferred.

(6) The present study provides quantitative correlations and design guidelines for passive flow rectifiers operating in the laminar-transitional regime. Future work should extend the analysis to fully turbulent flows, unsteady (pulsating) conditions, compressible flow regimes, and multiphase flows to cover a broader range of practical applications.

# References

[1] Park, H., & Kim, S. Y. (2026). Pressure drop characteristics of Tesla valve in fully turbulent flow. Journal of Fluids Engineering, 148(3).

[2] Tesla, N. (1920). Valvular conduit (U.S. Patent No. 1,329,559). U.S. Patent and Trademark Office.

[3] Han, Q., Liu, Z., Zhang, C., & Li, W. (2023). Enhance flow boiling in Tesla-type microchannels by inhibiting two-phase backflow. International Journal of Heat and Mass Transfer, 214.

[4] Forster, F. K., Bardell, R. L., Afromowitz, M. A., Sharma, N. R., & Blanchard, A. (1995). Design, fabrication and testing of fixed-valve micro-pumps. ASME International Mechanical Engineering Congress and Exposition.

[5] Truong, T. Q., & Nguyen, N. T. (2003). Simulation and optimization of Tesla valves. In Nanotechnology Conference and Trade Show (Nanotech 2003) (pp. 178-181).

[6] Zhang, S., Winoto, S. H., & Low, H. T. (2007). Performance simulations of Tesla microfluidic valves. In 1st International Conference on Integration and Commercialization of Micro and Nanosystems (pp. 15-19).

[7] Gamboa, A. R., Morris, C. J., & Forster, F. K. (2005). Improvements in fixed-valve micropump performance through shape optimization of valves. Journal of Fluids Engineering, 127(2), 339-346.

[8] Mohammadzadeh, K., Kolahdouz, E. M., Shirani, E., & Shafii, M. B. (2013). Numerical investigation on the effect of the size and number of stages on the Tesla microvalve efficiency. Journal of Mechanics, 29(3), 527-534.

[9] Nobakht, A. Y., Shahsavan, M., & Paykani, A. (2013). Numerical study of diodicity mechanism in different Tesla-type microvalves. Journal of Applied Research and Technology, 11(6), 876-885.

[10] Thompson, S. M., Paudel, B. J., Jamal, T., & Walters, D. K. (2014). Numerical investigation of multi-staged Tesla valves. Journal of Fluids Engineering, 136(8).

[11] Jin, Z. J., Gao, Z. X., Chen, M. R., & Qian, J. Y. (2018). Parametric study on Tesla valve with reverse flow for hydrogen decompression. International Journal of Hydrogen Energy, 43(18), 8888-8896.

[12] Nguyen, Q. M., Abouezzi, J., & Ristroph, L. (2021). Early turbulence and pulsatile flows enhance diodicity of Tesla macrofluidic valve. Nature Communications, 12(1).

[13] Thompson, S. M., Jamal, T., Paudel, B. J., & Walters, D. K. (2013). Transitional and turbulent flow modeling in a Tesla valve. In ASME International Mechanical Engineering Congress and Exposition.

[14] Yontar, A. A., Sofuoglu, D., Degirmenci, H., Bicer, M. S., & Ayaz, T. (2021). Investigation of flow characteristics for a multi-stage Tesla valve at laminar and turbulent flow conditions. Journal of Scientific Reports-A, (047), 47-67.

[15] Qian, J. Y., Wu, J. Y., Gao, Z. X., Wu, A. J., & Jin, Z. J. (2019). Hydrogen decompression analysis by multistage Tesla valves for hydrogen fuel cell. International Journal of Hydrogen Energy, 44(26), 13666-13674.

[16] Monika, K., Chakraborty, C., Roy, S., Sujith, R., & Datta, S. P. (2021). A numerical analysis on multi-stage Tesla valve based cold plate for cooling of pouch type Li-ion batteries. International Journal of Heat and Mass Transfer, 177.

[17] Lu, Y. B., Wang, J. F., Liu, F., et al. (2022). Performance optimization of Tesla valve-type channel for cooling lithium-ion batteries. Applied Thermal Engineering, 212.

[18] Bohm, S., Phi, H. B., Moriyama, A., et al. (2022). Highly efficient passive Tesla valves for microfluidic applications. Microsystems and Nanoengineering, 8(1).

[19] Purwidyantri, A., & Nguyen, T. A. D. (2023). Tesla valve microfluidics: The rise of forgotten technology. Chemosensors, 11(4).

[20] Shi, Y., Han, J., Zhang, B., & Li, W. (2026). Hydraulic-thermal characteristics of asymmetric Tesla valve microchannel. International Journal of Heat and Mass Transfer. Manuscript under review.

[21] Han, J., Shi, Y., Zhang, B., & Li, W. (2026). Flow boiling in parallel copper microchannels with asymmetric Tesla valves. Applied Thermal Engineering. Manuscript under review.

[22] Li, W., Yang, S., Chen, Y., Li, C., & Wang, Z. (2023). Tesla valves and capillary structures-activated thermal regulator. Nature Communications, 14.

[23] Qin, Z., & Wang, B. (2025). Design and diodicity enhancement mechanism of a double-baffle Tesla valve. International Journal of Heat and Mass Transfer, 239.

[24] Li, W., Luo, K., Li, C., & Joshi, Y. (2022). A remarkable CHF of 345 W/cm2 is achieved in a wicked-microchannel using HFE-7100. International Journal of Heat and Mass Transfer, 187.

[25] Qian, C., Wang, Y., Chen, Z., & Liu, H. (2025). Geometric optimization of a Tesla valve through machine learning to develop fluid pressure drop devices. Fluids, 10(10).

[26] Bardell, R. L. (2000). The diodicity mechanism of Tesla-type no-moving-parts valves (PhD thesis). University of Washington, Seattle, WA, USA.

[27] Truong, T. V., & Nguyen, N. T. (2004). Micromachined silicon Tesla valves. Sensors and Actuators A: Physical, 110(1-3), 126-132.

[28] Gamboa, A. R., Morris, C. J., & Forster, F. K. (2005). Improvements in fixed-valve micropump performance through shape optimization of valves. Journal of Fluids Engineering, 127(2), 339-346.

[29] Razavi, S. E., & Shirani, E. (2018). Numerical investigation of flow behavior in Tesla micromixers and valves. Chemical Engineering Research and Design, 132, 101-112.

[30] de Vries, S. F., Brouwers, H. J. H., & van der Geld, C. W. M. (2017). A Tesla-type valve for pulsating heat pipes. International Journal of Heat and Mass Transfer, 105, 1-11.

[31] Thompson, S. M., Ma, H. B., & Wilson, C. (2011). Investigation of a flat-plate oscillating heat pipe with Tesla-type check valves. Experimental Thermal and Fluid Science, 35(7), 1265-1273.

[32] Yang, K. S., Wang, C. C., & Tsai, P. H. (2019). Numerical optimization of Tesla valve structures for enhanced flow rectification. Applied Thermal Engineering, 148, 963-972.

[33] Kalsi, S., Jakhar, A., & Mankotia, K. (2022). Thermal-hydraulic performance of twisted tape inserts in flat tube radiators. Applied Thermal Engineering, 219, 119281. https://doi.org/10.1016/j.applthermaleng.2022.119281

[34] Kalsi, S., Jakhar, A., & Mankotia, K. (2023). Performance enhancement of solar thermal collectors using passive flow techniques. Solar Energy, 256, 04004. https://doi.org/10.1016/j.solener.2023.04.004

[35] Kalsi, S., Jakhar, A., & Mankotia, K. (2025). Novel insert geometries for enhanced heat transfer in automotive radiators. Applied Thermal Engineering, 262, 126769. https://doi.org/10.1016/j.applthermaleng.2025.126769